.=m5

.$1ArmOS

.$2DDS ARM Tests system

.s10

.c *********

.c * ArmOS *

.c *********

.s2

.c A microkernel

.c Operating Systems

.s2

.c for the

.s2

.c ARM processor

.s2

.c	and the

.s2

.c @2

.s20

.c Dave Dunfield

.c May 5, 2003

.s

.c Last updated: July 25, 2008

.t @1

.P

> INTRODUCTION

@1 is a small, simple and efficient operating system for the ARM processor.

@1 provides a number of system services, including:

.f-

- Multitasking kernel and process management services.

- Interrupt management and dispatching services.

- Real time event and timer services.

- Communication services between processes, interrupt

handlers and the operating system.

- Hardware control and arbitration services.

- System information collection and debugging services.

- Misc. other system servics.

.f+

There are two versions of the @1 operating system, which are conditionally

compiled:

>> Development version

This version provides extra debugging and analysis functionality, at a slight

execution time overhead cost. The additional functionality is enabled by

#define symbols in OS.H, normally these are ALL enabled in the development

version of @1:

.f-

OO_DEV:		Check that OS services are called in the proper context.

Check for OS_swap() in non-application context.

Check for corruption of the message free pools.

Check for various service indexes out of range.

Track message releases.

Automatically enable hardware memory protection.

Automatically enable the PANIC debugger.

OO_MLOW:	Track message free pool low-water marks.

OO_MHIGH:	Track process message queue high-water marks.

OO_TIME:	Track process/irq CPU time usage.

.f+

>> Release version

The release version of @1 is leaner and more efficient that the Development

version, however less debug and performance information is available. In the

release version, ALL of the symbols described above are normally turned OFF.

.p

> PROCESSES (also called Tasks)

Processes are program execution threads which operate concurrently and

independantly from each other under the control of operating system.

Any block of code within the ARM memory address can be started as an

independant process. Processes will automatically context switch to

ready/not-ready states as they block on various events (usually messages).

Processes in a blocked state will not consume any CPU resources until an

event occurs which causes the process to unblock. In addition, a process

can volentarily suspend its execution allowing other processess to run

by calling the entry point "OS_swap()" in this case, the process remains

ready and will resume execution the next time its "slot" reaches the

top of the ready queue.

The operating system does NOT automatically suspend processes and context

switch to new ones based on a time limit! - a process MUST block or call

OS_swap() on a regular basis. This avoids many potential synchronization

and timing problems which occur with involentary swapping, however it does

mean that all of our processes MUST be "well behaved". NOTE: Waiting for

a message or an event will automatically block - processes need explicitly

swap only when running an extended interval of processing, or within

polling loops.

The context of each process (registers, stack etc.) is saved when a

process context switches to the operating system, and is completely

restored when the process is next dispatched.

>> Process states

At any given time, a process is determined to be in one of the following

'states'. All states except TASK_READY indicate that the process is blocked

and not consuming CPU resources:

.f-

TASK_DEAD		0	= Never-started or has been killed

TASK_READY		1	= Actively executing

TASK_RECEIVE	2	= Blocked waiting for a message

TASK_MUTEX		3	= Blocked waiting for a Mutex

TASK_SIGNAL		4+	= Blocked waiting for signaled event

.f+

.p

>> Process Identification

All processes on the system are given a unique identifier (Pid), which

happens to simply be the index to the processes Task Control Block in the

system TCB table. Since TCBs are handed out on a "first available" basis,

this means that processes started during system initialization will have

deterministic Pid, which will simply increment for each process started.

Once a process terminates however, its Pid will be reused and this

determinism is lost.

It is anticipated that most of the administrative processes will

be started in a specific sequence during system initialzation and will never

terminate, therefore allowing other processess to communicate with them using

known fixed Pids. If messages must be sent to a process which may have

started after system initialization, it would be best to save its Pid in

a global variable so that sending process will be able to identify it.

(This is good practice even for deterministic Pids).

Process to process context switching is simplified if the OS can know that

it will always have a process to dispatch (ie: the ready list appears

"endless") For this reason, the system maintains a special "idle" process,

which runs at a reserved priority lower than all user processes. The idle

process never blocks, and never terminates, and is active any time there

is no other processes in the TASK_READY state. This process is also used

to perform several low-priority background system services.

Pid zero is reserved for the idle process. Application processes will

therefore begin with Pid=1. This also means that the value 0 can be used

as an Pid indicator that a transient process is not available.

>> Process Priorities

This OS uses a modified preemptive scheme - rather than having one priority

level completely block all lower priorities as is done in a standard

preemptive kernel, this system is structured such that when all the processes

at a given priority have run, one process from the next lower level runs and

then all processes at the current level run again. In other words, at a

given priority level, all lower priority processes take up one "slot" in the

time allocation for that priority level.

This scheme retains the idea that higher priority processes will have

(nearly) full use of the CPU while continuing to allow lower priority

processes to run at all times.

There are currently four priority levels, three "user" levels, and a

fourth reserved level which is available only to the "idle" process (this

special priority list links to itself so that the priority list never ends,

and the system never tries to locate a lower priority level).

.P

> KERNEL SERVICES

@1 provides a number of kernel services which perform system related functions

involving the various subsystems which are controlled by the operating system.

The system services entry points are defined as variable parameter functions,

which allows any argument types to be presented, however the 32 bit return

type (when applicable) defaults to "unsigned", and may need to be casted to

the appropriate return value type required by the requesting client.

>> Calling Modes

System services are performed within the ARM privileged context of the

operating system. Depending on the context of the calling program, there

are three ways to access most system services:

.i+3

.i-3 U: Calls made by user mode application programs (processes) all occur

through a single "OS_call" entry point. This performs a context switch into

kernel privileged mode before processing the request.

The first argument will always be an "OS_...." constant which indicates

the OS service being requested (please refer to OS.H for values).

"OS_call" must NOT be called within an interrupt handler! - Call from

mainline application programs (process) only.

.i-3 I: These are a number of entry points which go directly to the

operating systems internal kernel service function. These must be called in

privileged mode, and therefore must NOT be called from a mainline application

program (process) - Call from within the context of an INTERRUPT only!

.i-3 D: These are "dual mode" entry points which determine the correct method

of requesting the service by examining the current ARM state. If running as

an mainline application program, they will perform the OS_call(...). If running

as interrupt, they will call the direct (interrupt) entry point.

These function are provided to accomodate code which is common to both

mainline application programs and interrupt handlers however it is a more

"expensive" way to call the OS that the other two means - Although it is

tempting to simply use the D calls and not worry about it, it is preferable

to use the appropriate U or I calling mode in situations where the code

always executes in one particular context.

.i-3

Note that not all calling modes are supported by each service. If a particular

calling mode (U, I or D) is not listed under a service, than that calling mode

is not available for that service.

.t @1 - Process management services

.P

> KERNEL SERVICES INDEX

Here are the kernel services which are currently available within @1:

.f-

>> Process management services

.i-

OS_PROC_START											  OS_PROC_START

U: unsigned OS_call(OS_PROC_START, (*Function)(), unsigned Priority,

unsigned *Stack, unsigned Ssize, unsigned char name,

unsigned Mqlimit)

Parameters:	Priority	= Priority level of process (0-#priorities)

Function	= Address of function to execute

Stack		= Low bound of process stack memory

Ssize		= Size of process stack (in words)

Name		= Pointer to 8-character process name

Mqlimit		= Limit on # of messages which can be

queued on this process.

Returns:	Process ID of new process.

.f+

Starts a process executing. A TCB will be established and filled in for

the process, which will begin running at the specified Function address.

.f-

.p

OS_PROC_UNREADY											OS_PROC_UNREADY

U: void OS_call(OS_PROC_UNREADY, unsigned Pid, unsigned State)

Parameters:	Pid			= Process id of process

State		= New state value

Returns:	nothing

.f+

Removes a process from the ready list and sets a new (non-executing)

state.

If process is not ready, it will change state only.

Setting State=TASK_DEAD will kill the process

To commit suicide (terminate current task), use:

.c OS_call(OS_PROC_UNREADY, OS_active_tcb->Id, TASK_DEAD)

Do NOT use State=TASK_READY (illegal!)

.f-

.p

OS_PROC_READY											  OS_PROC_READY

U: void OS_call(OS_PROC_READY, unsigned Pid, unsigned Value)

I: void OS_proc_ready(unsigned Pid, unsigned Value)

Parameters:	Pid			= Process id to make ready

Value		= Value passed back to unblocked task

Returns:	nothing

.f+

Returns a process to ready execution state.

Normally used to restart a process which has blocked pending an event

when that event has occured (this is often done within interrupt handlers).

.f-

.p

OS_PROC_WAIT											   OS_PROC_WAIT

U: unsigned OS_call(OS_PROC_WAIT, unsigned Event)

Parameters:	Event		= Event ID to wait for (<MAX_SIGNAL)

Returns:	Value passed back from event source

.f+

Waits for a specific system event. The calling process blocks until

the specified event is signaled (see OS_PROC_SIGNAL).

.f-

.p

OS_PROC_SIGNAL											 OS_PROC_SIGNAL

U: void OS_call(OS_PROC_SIGNAL, unsigned Event, unsigned Value)

I: void OS_proc_signal(unsigned Event, unsigned Value)

D: void SOS_proc_signal(unsigned Event, unsigned Value)

Parameters:	Event		= Event ID to signal (<MAX_SIGNAL)

Value		= Value to pass back to waiting processes

Returns:	nothing

.f+

Signals the occurance of a system event.

All waiting processes are unblocked and returned to the ready state.

The Value passed is returned to each waiting process (This is essentially

a one-to-many broadcast).

.f-

.t @1 - Interprocess messaging services

.p

>> Interprocess messaging services

.i-

OS_MSG_SMALL											   OS_MSG_SMALL

OS_MSG_MEDIUM											  OS_MSG_MEDIUM

OS_MSG_LARGE											   OS_MSG_LARGE

U: struct MESSAGE *OS_call(OS_MSG_SMALL)

U: struct MESSAGE *OS_call(OS_MSG_MEDIUM)

U: struct MESSAGE *OS_call(OS_MSG_LARGE)

I: struct MESSAGE *OS_msg_small()

I: struct MESSAGE *OS_msg_medium()

I: struct MESSAGE *OS_msg_large()

D: struct MESSAGE *SOS_msg_small()

D: struct MESSAGE *SOS_msg_medium()

D: struct MESSAGE *SOS_msg_large()

Parameters:	none

Returns:	Address of allocated message block

.f+

Allocates a Small (128 byte), Medium (512 byte) or Large (1536 byte)

message block.

The system is highly dependant on messages It cannot run without available

message blocks. These calls will cause an OS_panic() exception if no suitable

message blocks are available. NOTE: The system will automatically attempt to

allocate a larger block if no blocks of the requested size are available.

Applications requiring a large number of messages blocks should check the

OS_free_msg_* indicators to avoid exhausting the message pools.

.f-

.p

OS_MSG_RELEASE											 OS_MSG_RELEASE

U: void OS_call(OS_MSG_RELEASE, struct MESSAGE *Msg)

I: void OS_msg_release(struct MESSAGE *Msg)

D: void SOS_msg_release(struct MESSAGE *Msg)

Parameters:	Msg			= Address of block to release

Returns:	nothing

.f+

Releases a message block back to the OS free message pool. All application

software must take care that all allocated messages are properly released

to avoid exhausting the message pools (which will cause a system panic).

.f-

.p

OS_MSG_SEND													OS_MSG_SEND

U: unsigned OS_call(OS_MSG_SEND, unsigned Pid, struct MESSAGE *Msg)

I: unsigned OS_msg_send(unsigned Pid, struct Message, *Msg,

unsigned char Sender);

D: unsigned SOS_msg_send(unsigned Pid, struct Message, *Msg,

unsigned char Sender);

Parameters:	Pid			= Process id to receive message

Msg			= Previously allocated message block

Sender		= Sender ID (interrupt only)

Returns:	0			= Message was delivered directly to waiting proc.

1			= Message was queued for busy proc.

2			= Process is TASK_DEAD - cannot receive message

3			= Process id is invalid.

4			= Process message queue fill.

.f+

Sends a message to the process identified by Pid. If the process is receive

blocked, it will be made ready and the message passed directly to it,

otherwise the message is queued for later delivery.

Sender must allocate message with OS_MSG_SMALL/MEDIUM/LARGE then fill it in

before OS_MSG_SENDing.

On a successful send, ownership of the message block passes to the receiving

process. The sender must NOT perform any further manipulation of the message.

The receiver must OS_MSG_RELEASE the message block when it has finished

processing the content.

If a message cannot be sent, ownership of the message block remains with the

sender, who must OS_MSG_RELEASE it if there is no further use for the block.

.f-

.p

OS_MSG_RECEIVE											 OS_MSG_RECEIVE

U: struct MESSAGE *OS_call(OS_MSG_RECEIVE)

Parameters:	none

Returns:	Address of received message block

.f+

Receive a message.

This call checks for queued messages which have been sent to the current

process. If no messages are available, the process blocks and waits for a

message to be sent during which time the process is removed from ready

execution.

If messages exist in the process receive queue, the oldest one is removed

from the queue and returned to the process which does NOT block.

.f-

.p

OS_MSG_CHECK											   OS_MSG_CHECK

U: struct MESSAGE *OS_call(OS_MSG_CHECK)

Parameters:	none

Returns:	0	= No message available

!0	= Address of received message block

.f+

Conditonally receive a message.

This call DOES NOT BLOCK.

.f-

.p

OS_MSG_REQUEUE											 OS_MSG_REQUEUE

U: void OS_call(OS_MSG_REQUEUE, struct MESSAGE *Head, struct MESSAGE *Tail)

Parameters:	head	= Pointer to first message in list

tail	= Pointer to last message in list

Returns:	nothing

.f+

Requeue a block of messages previously removed from the input qieie.

This function provides a means of waiting for a specific message, by locally

queueing undesired messages, and then returning them to the input queue when

the desired message is found:

.f-

struct MESSAGE *m, *head, *tail;

// ...

// Wait for a TIMER message (info==255)

// Save and requeue non-timer messages

head = 0;					// Initial state (no queue)

for(;;) {

m = (struct MESSAGE*)OS_call(OS_MSG_RECEIVE);

if(m->Info == 255)	// Timer: This is what we want

break;			// so... Exit wait loop

if(head) {

tail->Link = m;	// Link to previous tail

tail = m;		// and adjust tail to current

continue; }

head = tail = m; }	// First time - init head/tail

if(head)

OS_call(OS_MSG_REQUEUE, h, t);	// Requeue local messages

// ... process and release message 'm'

// ... Next time we call OS_MSG_RECEIVE, we will receive

// ... the "pending" messages which were requeued.

.t @1 - Timer services

.p

>> Timer services

.i-

OS_TIMER_START											 OS_TIMER_START

U: unsigned OS_call(OS_TIMER_START, unsigned Init, unsigned Repeat,

(*Function)(), unsigned Parmameter)

Parameters:	Init		= Initial interval (1.25ms units)

Repeat		= Repeat interval  (1.25ms units)

Function  0 = Send me a message every interval

!0 = Call this function every interval

Parmameter	= User timer parameter

Returns:	Handle value used to reference timer (0-#timers)

.f+

Establishes a process timer.

"Init" value is time to first timer tick.

"Repeat" value is time to subsequent timer ticks. Setting Repeat=0 indicates

a single-shot timer which is automatically stopped after Init interval

expires.

Timer message indicates: Info=255, Info1 = timer handle, Data[0-3] =

Parameter (32 bit value).

Call back Function is executed in a IRQ privileged context using the ARM IRQ

stack - this places restrictions on OS services available in callbacks.

Parameter. is passed as single 32 bit parameter to the callback function.

.f-

.p

OS_TIMER_STOP											  OS_TIMER_STOP

U: unsigned OS_call(OS_TIMER_STOP, unsigned Handle)

Parameters:	Handle		= Timer handle returned by OS_TIMER_START

Returns:	nothing

.f+

Stops a previously established timer.

NOTE: Timer may fire (send message or execute callback function) between

the interval when you call this service and it returns.

.f-

.p

OS_TIMER_RESET											 OS_TIMER_RESET

U: unsigned OS_call(OS_TIMER_RESET, unsigned Handle, unsigned Interval)

Parameters:	Handle		= Timer handle returned by OS_TIMER_START

Interval	= New timer interval value for timer

Returns:	Nothing

.f+

Resets the current timeout interval for an active timer. The timer will next

fire after "Interval" ticks of the system clock occur. Any interval currently

being timed by the timer is discarded and replaced by the new interval.

This is useful to reset a timer to a known timeout interval without the

overhead of stopping and restarting it.

NOTE1: It is illegal to reset a timer which is not active - therefore to

to avoid the possibility that the timer you are about to reset may have

expired and automatically deactivated, you should NOT use this service

with single-shot timers. Instead, define a repeating timer, and explicitly

stop it when it expires.

NOTE2: This affects the current timing interval only - does not affect the

repeat value. Once the set interval expires, the timer will automatically

load the repeat value as its next interval.

.f-

.t @1 - Interrupt management services

.p

>> Interrupt management services

.i-

OS_IRQ_DISABLE											 OS_IRQ_DISABLE

OS_IRQ_ENABLE											  OS_IRQ_ENABLE

U: void OS_call(OS_IRQ_DISABLE)

U: void OS_call(OS_IRQ_ENABLE)

Parameters:	none

Returns:	nothing

.f+

Disables or Enables ALL processor interrupts.

Disabling interrupts stops all timers and any hardware which is dependant on

interrupts. It is provided to allow user mode programs to impliment semiphores,

locks and other constructs requiring exclusive control of the CPU.

Since this OS does not task-swap unless a process blocks or gives up its

timeslice, this function is not normally needed except in cases where an

interrupt triggered handler or callback function is involved.

Call OS_IRQ_ENABLE as SOON AS POSSIBLE after invoking this service.

.f-

.p

OS_IRQ_SET													 OS_IRQ_SET

OS_IRQ_CLEAR											   OS_IRQ_CLEAR

U: void OS_call(OS_IRQ_SET, unsigned Mask)

U: void OS_call(OS_IRQ_CLEAR, unsigned Mask)

I: vois OS_irq_set(unsigned Mask);

I: vois OS_irq_clear(unsigned Mask);

D: vois SOS_irq_set(unsigned Mask);

D: vois SOS_irq_clear(unsigned Mask);

Parameters:	Mask		= Interrupts to change (See HWNOTES.TXT)

Returns:	nothing

.f+

Enable (set) or Disable (cleaR) specific hardware interrupts.

See IRQ.H for IRQ sources & mask values

.f-

.t @1 - Console management services

.p

>> Console management services

.i-

OS_CONSOLE_TX											  OS_CONSOLE_TX

U: unsigned OS_call(OS_CONSOLE_TX, struct MESSAGE *Msg)

I: unsigned OS_console_tx(struct MESSAGE *Msg)

D: unsigned SOS_console_tx(struct MESSAGE *Msg)

Parameters:	Msg			= Message containing data to send

(Info1 indicates length of block)

Returns:	0	= Message has been queued for output

!0	= Message lost due to console allocation

.f+

Sends data to the system console.

Message is queued for console serial-TX interrupt handler.

Processes with a lot of serial output should monitor OS_free_msg_* and

regulate their output when the free message pool nears depletion.

If console has been allocated for exclusive use by another task, output

will be thrown away.

If console is NOT allocated to another task, messages will be output in

the order in which they were queued.

This is a BINARY service - OS does NOT expand '\n' to include '\r' - use

"\r\n" to output a new line.

Once this call has been made, ownership of the message passes to the OS -

the OS will release the message when it has been transmitted.

.f-

.p

OS_CONSOLE_RX											  OS_CONSOLE_RX

U: unsigned OS_call(OS_CONSOLE_RX, unsigned char *Buffer, unsigned Length)

Parameters:	Buffer		= Buffer to receive console data

Length		= Maximum length to transfer (1-2048)

Returns:	0-2048 		= Number of characters received

0xFFFFFFFF	= Console not allocated (see below).

.f+

Test for data from system console

Console must be allocated (see OS_CONSOLE_ALLOC) before you can request

console input data.

It is OK to request a Length of >2048 charcters, however the OS will return

at most 2048 characters which is the size of the console receive buffer.

.f-

.p

OS_CONSOLE_ALLOC									   OS_CONSOLE_ALLOC

U: unsigned OS_call(OS_CONSOLE_ALLOC)

Parameters:	none

Returns:	0		= Console has been allocated

!0		= Another process already has it allocated

.f+

Allocate console to calling task.

Console must be allocated in order to receive data.

Well behaved processes should NOT attempt output to the console when it

is allocated by another process (this data will be discarded).

Allocating the console blocks all other tasks from performing console I/O.

.f-

.p

OS_CONSOLE_RELEASE									 OS_CONSOLE_RELEASE

U: unsigned OS_call(OS_CONSOLE_RELEASE)

Parameters:	none

Returns:	0	= Console released

!0	= Another task owns the console (was not owned)

.f+

Release system console.

Release the console as soon as possible after allocating.

.f-

.pe

OS_CONSOLE_AT											  OS_CONSOLE_AT

U: void OS_call(OS_CONSOLE_AT)

Parameters:	none

Returns:	Nothing

.f+

Allocate process to receive "AT|" commands.

Once this call is executed, any commands on the system console beginning

with "AT|" will be sent to the calling process as a message.

.f-

.t @1 - System control services

.p

>> System control services

.i-

OS_SYS_STOP													OS_SYS_STOP

U: void OS_call(OS_SYS_STOP)

Parameters:	none

Returns:	nothing

.f+

Terminate operating system & all processes.

Use for debugging / exceptional situtions only.

Shuts down operating system and all processes.

Disables all interrupts

Returns to caller in supervisor state

The only service which works after this is: OS_SYS_RESTART

.f-

.p

OS_SYS_RESTART											 OS_SYS_RESTART

U: void OS_call(OS_SYS_RESTART)

Parameters:	none

Returns:	never returns

.f+

Restart operating system.

Implements a cold restart of the system

.f-

.t @1 - Ethernet Services

.p

>> Ethernet Services

.i-

OS_ETHERNET_MAC											OS_ETHERNET_MAC

U: unsigned char *OS_call(OS_ETHERNET_MAC)

Parameters:	None

Returns:	Pointer to 6 byte Network address, 0 if no network

.f+

Obtains ethernet hardware address.

This function should be used to obtain the MAC address which is placed in

the "from" address field of any packets transmitted via OS_ETHERNET_TX

.f-

.p

OS_ETHERNET_TX											 OS_ETHERNET_TX

U: unsigned OS_call(OS_ETHERNET_TX, unsigned char *Data, unsigned Size)

I: unsigned OS_ethernet_tx(unsigned char *Data, unsigned Size)

D: unsigned SOS_ethernet_tx(unsigned char *Data, unsigned Size)

Parameters:	Data	= Packet data to transmit

Size	= Size of packet data to send (including header)

Returns:	1		= Packet was transmitted immediately

2		= Packet was queued for transmission

0		= Packet was NOT transmitted (network congestion)

.f+

Sends an ethernet packet.

Data should contain this packet structure:

.f-

struct PACKET {

unsigned char	to[6];		// Destination address

unsigned char	from[6];	// Source (our) address

unsigned short	type;		// Packet type

unsigned char	data[Size-14];	// Packet data

};

.f+

This is a direct call to the Ethernet driver - when it returns, the packet

has been sent.

.f-

.p

OS_ETHERNET REGISTER							   OS_ETHERNET_REGISTER

U: void OS_call(OS_ETHERNET_REGISTER, unsigned Type)

Parameters:	Type	= Packet type to receive (16 bit, > 1536)

Returns:	Nothing

.f+

Registers interest in a packet type.

After performing this function, any ethernet packets received with the

indicated type will be sent to the requesting process as a standard OS

message containing the packet structure as its data with a sender (Info)

indicating a value of 254. Info1 contains size of data.

If two tasks request the same type, or the packet routing table becomes

exhausted, an OS_panic() will result (this should never happen in our

system).

.f-

.p

OS_ETHERNET_RELEASE									OS_ETHERNET_RELEASE

U: void OS_call(OS_ETHERNET_RELEASE, unsigned Type)

Parameters:	Type	= Packet type to release

Returns:	Nothing

.f+

Releases interest in a packet type.

After performing this function, no further messages will be send by the

ethernet subsystem for this packet type.

.f-

.t @1 - Mutex services

.p

>> Mutex Services

.i-

OS_MUTEX_REQUEST									   OS_MUTEX_REQUEST

U: unsigned OS_call(OS_MUTEX_REQUEST, unsigned Mutex, unsigned Request)

Parameters:	Mutex	= Mutex desired: 0 - (MAX_MUTEX-1)

Request	=	0 = No action if Mutex unavailable

1 = Send message when Mutex assigned

2 = Block until Mutex assigned

Returns:	0		= Mutex is unavailable

1		= Mutex has been assigned (immediately)

2		= Mutex has been assigned (after blocking)

.f+

Request access to a Mutex

For all Request types, return 1 indicates that Mutex was available and

assigned immediately - no further action is taken.

For Request==1, return 0 indicates Mutex is unavailable. OS will queue request

and assign Mutex when it becomes available & send message: (Info=253,

Info1=Mutex) to inform requester that Mutex has been assigned.

For Request==2, return 2 indicates that Mutex was NOT available at time of

request, and that task was blocked until Mutex became available.

.f-

.p

OS_MUTEX_RELEASE									   OS_MUTEX_RELEASE

U: void OS_call(OS_MUTEX_RELEASE, unsigned Mutex)

Parameters:	Mutex	= Mutex being released: 0-(MAX_MUTEX-1)

Returns:	Nothing

.f+

Release a previously allocated Mutex

.f-

.t @1 - System hardware services

.p

.f-

.t @1 - Storage services

.p

>> Storage Services

.i-

OS_STORE_READ											  OS_STORE_READ

U: unsigned OS_call(OS_STORE_READ, unsigned Device, unsigned Sector,

unsigned char *Address)

Parameters:	Device		= 0=Internal Flash, !0=SmartMedia card

Sector		= 512 byte "Sector number" to read

Address		= Memory address to receive data

Returns:	0			= Success

1			= Device is busy - please retry later

0xFFFFFFFF	= Unrecoverable error reading sector

.f+

Read data from permanant storage.

Pages are fixed at 512 bytes in size.

Each 256 byte half page is protected by a single bit error recovery scheme.

.f-

.p

OS_STORE_WRITE											 OS_STORE_WRITE

U: unsigned OS_call(OS_STORE_WRITE, unsigned Device, unsigned Sector,

unsigned char *Address)

Parameters:	Device		= 0=Internal Flash, !0=SmartMedia card

Sector		= 512 byte "Sector number" to write

Address		= Memory address contining data

Returns:	0			= Success

1			= Device is busy - please retry later

.f+

Write data to permanent storage.

Pages are fixed at 512 bytes in size.

Flash memory already contining data !=0xFF cannot be overwritten without

erasing the block containing this sector.

Due to ECC code and above limitation, you cannot write a partial sector!

.f-

.p

OS_STORE_ERASE											 OS_STORE_ERASE

U: unsigned OS_call(OS_STORE_ERASE, unsigned Device, unsigned Block)

Parameters:	Device		= 0=Internal Flash, !0=SmartMedia card

Block		= Block number to erase

Block 0 contains sectors  0-31

Block 1 contains sectors 32-63

Block 3 contains sectors 64-95 ...

Returns:	0			= Success

1			= Device is busy - please retry later

.f+

Erase a 32 sector "block" of flash.

You MUST erase a block before you can write to sectors within that block.

Once written, you cannot overwrite sectors until the block is erased.

.f-

.t @1 - Misc. services

.p

>> Misc. services

.i-

OS_PANIC_REGISTER									  OS_PANIC_REGISTER

U: void OS_call(OS_PANIC_REGISTER, unsigned char *name, void *func)

Parameters:	Name	= Name of extended command (UPPERCASE, max 8 chars)

Func	= Address of function to address

Returns:	Nothing

.f+

Register an extension to the PANIC debugger.

Adds a command to the PANIC debugger. When the named command is entered, the

address will be called, and receive as a single argument the address of the

console command pointer. You can use the library functions: parse() and

parse_number() to obtain command arguments.

.f-

.t @1 - System support services

>> System support services

.i-

SS_INV_TLB													 SS_INV_TLB

U: void OS_call(OS_SYS_SERV, SS_INV_TLB);

Parameters:	SS_INV_TLB	= Function request

Returns:	Nothing

.f+

This function invalidates the Arm Memory Management Unit Translation Lookaside

Buffers - this forces the ARM to reread and recache these entries when

resolving virtual memory accesses (Required when memory mapping or protection

TLB entries have been changed).

This function should be used ONLY by OS support services running in user mode

(ie: NOT by application software).

.t @1

.p

> OTHER ENTRY POINTS:

In addition to the kernel services entry point "OS_call()", the following

other entry points are provided:

>> Application entry points

The following OS entry points may be used by all applications:

.f-

void OS_swap(void)

Parameters:	none

Returns:	nothing

.f+

Gives up remainder of current task execution slot.

.i Allows other tasks to run without blocking current task.

.i DO not call within an interrupt handler!

.f-

void OS_panic(char *Format, ...)

Parameters:	Format		= format specifier string, may contain:

...			= Additional operands as required by Format

Returns:	Nothing (See note)

.f+

Use to log an unrecoverable system event.

.i Event text is formatted in 'printf' style.

.i If debug is enabled, enters an interactive debugger, otherwise restarts

the sytem.

.i+

When debug is enabled, the 'CONT' command can be used to continue

from the OS_panic() call (ie: return from the function).

.p

> OS INTERNALS

>> Data Strcutures

The following operating system data structures may be of interest to

application programs:

>>> Task control block

.f-

struct TCB {

unsigned 		*Sp;			// Task stack pointer

struct TCB 		*Next;			// Link to next task (0 if end)

struct MESSAGE	*Mqueue;		// Pointer to message queue (0=no msgs)

unsigned char	*Name;			// Pointer to task name

unsigned char	Id;				// Task ID

unsigned char	State;			// Task state

unsigned char	Priority;		// Task priority

unsigned		Mqueue_size;	// Number of messages currently queued

unsigned		Mqueue_limit;	// Task message queue maximum limit

};

.f+

>>> Message block

.f-

struct MESSAGE {

struct MESSAGE	*Link;			// Link to next block (0 if end)

unsigned char	Info;			// Sending Tid / Message information

unsigned char	Info1;			// Misc. information byte

unsigned short	Info2;			// Misc. information word

unsigned char	Data[MSG_SIZE];	// Data block

};

.f+

>> Global variables

The following global variables within the OS have been made public,

and may be referenced by any process to determine information more

quickly than would be possible if a kernel service call were used.

NOTE that user processes must NEVER WRITE to these variables or any

memory area pointed to by them.

.i+

.i- unsigned char OS_hw_type;

Identifies hardware platform (useful if different platforms require different

driver action etc.)

.i- unsigned OS_system_config;

Contains the system configuration as determined at startup.

See CONFIG_* defintions in OS.H for bits and fields.

This gives the configuration as of system powerup. It does not give

current config for removable items (such as memory card).

.p

.i- unsigned OS_ram_top;

Contains the amount of RAM in the system, which ranges from

00000000-(OS_ram_top-1).

.i- struct TCB *OS_active_tcb;

Pointer to currently executing task control block.

A process can reference this to determine its Id, Priority, Name etc.

Neither the pointer or the structure to which it points may be modified!

.i- unsigned OS_free_msg_small;

.i- unsigned OS_free_msg_medium;

.i- unsigned OS_free_msg_large;

Indicates the number of free message blocks of the corresponding size

which are currently available in the system.

Can be used by processes which require a large number of messages to

insure that they do not consume message blocks faster than the system

can make them available.

.i- unsigned OS_timer_tick;

Begins at 0 when the OS boots, and increments every timer interrupt

(currently 1.25 millisecond ticks).

.i- unsigned OS_eirq_mask;

Contains the current enabled external hardware interrupt mask.

.i- unsigned OS_debug_mask;

Contains a 32 bit mask which is ANDed against the mask value of the

Dprintf() library function to determine which informational messages

should be displayed.

.i- unsigned char OS_console_owner;

Indicates current owner of console:

0	= indicates all processes may TX.

!0	= Pid of current system console owner.

.p

.i- unsigned char OS_at_owner;

Indicates currently registered AT command handler:

0	= No AT comamnd handler installed

!0	= Pid of AT command handler

.i- unsigned char OS_irq_active

Indicate state of the interrupt system:

0	= No interrupt active (mainline code)

!0	= Interrupt in progress

.i- unsigned OS_hw_port;

Contains current value (mirror) for the hardware diagnostic port

on the expansion module which is used for system timing analysis

(with oscilloscope).

.i-

.P

> CONSOLE COMMAND PROCESSOR

Once the system is operational, the low-level "OS" task evolves into a

simple console command processor. At the prompt, the following keys are

usable:

.f-

<ascii>		- All printable ASCII characters may be entered into

the text of a command to be executed

Return		- Execute the command

BackSpace	- Remove last entered character / correction.

ESC			- Redraws the entire command line - this is useful if

informational messages from other processes occurs

during command entry.

Control-C	- Cancel command input and return to fresh prompt.

.f+

>> Console commands

The following console mode commands are available:

>>> DEBUG debug-output-mask

Sets the value of OS_debug_mask, which controls which informational

messages are displayed on the the console.

>>> DUMP address [end-address]

Displays the content of system memory in Hex/Ascii dump format.

If end-address is not specified, default is: address+255

.p

>>> HALT

Performs a call to OS_panic(), which stops the system and starts the

panic debugger.

>>> INFO

Displays information about certain system global variables and

parameters (See Globals section):

.f-

ArmOS - Build: (build-date/time) [(Development)]

Hardware Type      : OS_hw_type

System config      : OS_system_config

Free message blocks: OS_free_msg_small/OS_free_msg_medium/OS_free_msg_large

Lowest free seen   : low-water marks for small/medium/large free pools *1

Timer tick         : OS_timer_tick

External IRQ mask  : OS_eirq_mask

Debug output mask  : OS_debug_max

Latency/max        : 0/1

Top of RAM         : OS_ram_top

S/I stack avail.   : amount of free space on SVC/IRQ stacks.

.f+

*1 - message pool low-water marks are available in the development version

of the OS only.

>>> PACKET [type]

Displays MAC address and packet_type->process registrations.

If [type] is specified, it will be registered to the console process

so that any ethernet packets of that type that are received will be

dumped to the console.

Following the packet_type->process registrations, the following

network driver statistics will be displayed:

.f-

TXok	: Number of successfully transmitted packets

TXque	: Number of times transmit packet had to be queued

TXdrop	: Number of dropped transmit packets (queue full)

RXok	: Number of successfully received packets

RXfail	: Number of receive packets aborted by NIC (error)

RX2big	: Dropped receive packets - too large to accept

RXnmsg	: Dropped receive packets - OS messages too low

RXnorx	: Dropped receive packets - Type not registered

.f+

>>> POKE address [data...]

Stores data bytes into memory beginning at the specified address (This is

VERY dangerous unless you know EXACTLY what and where you are writing!)

.p

>>> PS [name] [/H /M /R /I /T /N /C]

If [name] is specified, only processes with names beginning with the

specified string are shown. Otherwise all processes are displayed.

The /H option inhibits the output of a header (title).

With no other options, displays a list of all active processes in the

system:

.f-

Pid	Name State Priority Tcb Stack Sptr Sfree Mqueue Link

Name	= Name of task given at OS_PROC_START

State	= Current operating state of task (see OS.H)

Priority= Priority assignment for task

Tcb		= Address of task control block structure

Stack	= Task base stack block address

Sptr	= Tasks current stack pointer

Sfree	= Number of unused words in the stack block

Mqueue	= Pointer to messages queued for task (0=None)

Link	= Address of next Tcb in priority ready list

.f+

When /M is specified, displays process message queues:

.f-

#	Name	Current	Highest	Limit

#		=	Pid for process

Name	=	Process name

Current	=	Number of msgs current queued

Highest	=	Maximum size queue has reached

Limit	=	Mqlimit (See OS_PROC_START)

.f+

When /T is specified, displays real-time analysis data for each process:

.f-

#  Name    MaxSlot    --- History 1-7 ----

MaxRun  -------------- History 8-15

#			= Pid for process

Name		= Process name

MaxRun		= Maximum time seen during single dispatch

History		= Time used in last 15 "slots"

MaxSlot		= Maximum time taken in any slot.

All timing is done in 2us units.

Slot time is controlled by OS_RT_MASK in OS.H, and is in

units of 128ms. (Default 64 = 8.192s)

.f+

.p

When /I is specified, displays real-time analysis for each interrupt:

.f-

#  Name    MaxSlot    --- History 1-7 ----

MaxRun  -------------- History 8-15

#			= Interrupt index

Name		= Interrupt name

MaxRun		= Maximum time seen during single interrupt

History		= Time used in last 15 "slots"

MaxSlot		= Maximum time taken in any slot.

All timing is done in 2us units.

Slot time is controlled by OS_RT_MASK in OS.H, and is in

units of 128ms. (Default 64 = 8.192s)

.f+

The /T and /I options can be combined to obtain a single process + IRQ

display.

The /R can be added to /M, /T and /I to reset the "Maximum seen" values.

The /N option enables broadcast of real-time statistics over the Network

(/T and /I to the console are disabled during network broadcast). The /C

option disables network broadcast and return to Console operation.

/M /T /I /R /N and /C are available in the development version of the OS

only.

>>> SIGNAL event value

Signals a system event, passing specified value.

>>> TX type string

Broadcasts an ethernet packet of the indicated type with the remainder

of the command line as the packet data payload.

>>> READ <sector> [+]

Reads and displays the indicated bulk storage (Flash) sector (Only

the first 256 bytes are displayed, to see the last half, add '+'.

.p

>>> WRITE <sector> [data ...]

Writes the indicated bulk storage (Flash) sector with the indicated

byte values, padded to the 512 byte sector size with 0xFF.

>>> ERASE <page>

Erases the indicated bulk storage (Flash) block.

>>> PROTECT [/D]

Enables memory access protection in the release version of the operating

system. Once this command is issued, attempts to write the code space, or

to access unimplimented memory will cause an exception and system panic.

The /D option also enables the panic debugger. When a system panic occurs,

the interactive debugger will be automatically entered.

Memory protection and the panic debugger are ALWAYS enabled in the

development version of the operating system.

>>> <name> parmeter-string

Any process which has a name containing all upper-case characters

can be sent text messages from the console. The remainder of the

command line is copied into a message block and transmitted to the

process via OS_MSG_SEND.

.p

>> Console process notes

Any messages which are received by the console command processor will be

displayed on the console in memory dump format.

If another task allocates the console (OS_CONSOLE_ALLOC), the console

command processor will be unable to accept interactive input until the

console is released.

If information output occurs from another source while you are entering

a command, the command line can be restored by pressing ESCAPE.

.P

> STARTUP

On powerup reset or cold boot restart, the operating system performs

the following actions:

Initializes the ARM system control registers, memory config etc.

Initializes the other hardware

Initializes Supervisor, IRQ and one user mode stack.

Initializes OS internal data structures

Detects optional hardware configuration.

Initializes the bulk storage and ethernet subsystems

Establishes the multitasking environment

Establishes a 1ms timer interrupt

If enabled, re-initializes the ARM MMU to assert write protection on

the code memory, and read/write access protection on all unimplemented

memory areas.

Switches to user mode and launches the function "user_init" to

initialize the user application. This function must start any

processes and timers required by the application and then return.

.i NOTE: "user_init" runs in the context of the system "idle" task,

and must NOT block (see notes below).

When "user_init" returns, the thread evolves into the system console

command processor (CCP). If no CCP is desired, "user_init" may remain

active as the system "idle" loop, however it must follow these rules:

.i+

Since it is running as the IDLE task, 'user_init' must NEVER BLOCK!

It should never wait for messages (it is OK to check for them, and

to send them).

The traditional operating system implementation of an idle loop is

to simply dispatch the next process:

.f-

for(;;)

OS_swap();

.f+

However this task may perform any desired low priority background

activity as long as it NEVER BLOCKS! (this is important - the system

will fail if the "idle" process attempts to block!)

.i-

.P

>INTERRUPT HANDLERS

The following external interrupts may be enabled via th OS_IRQ_SET system

call, and will be directed to handlers within the application:

Note that these external IRQ handlers should service the interrupt, and

reset any hardware needed to clear the pending interrupt signal (otherwise

the interrupt will be reentered as soon as it returns.

.f-

.i+

.i- void IRQ_co_ring(unsigned Irq_mask)

Irq_mask	= indicates which CO's are enabled & ringing

CO line ring detector.

See IRQ.H for Irq_mask value

.i-

.f+

.p

> RESERVED SYMBOLS

All operating system entry points and public data areas use the

convention that symbol names will begin with "OS_".

Most of the OS code is contained within a single module, and uses

static global symbols wherever possible, there are however a few

"private" global names that I must employ between the main C code

module and assembly language blocks in the startup module. These

"private" items use the convention that symbol names will begin

with "__OS".

Please note that these two prefixes "OS_" and "__OS" are reserved for

exclusive use by the operating system. Furthermore, access to any global

area identified with a symbol beginning with "__OS" is to be avoided at

the application level.

Unless otherwise noted, any public OS data structure (those beginning

with "OS_" are to be treated as READ ONLY by the application).

> OS CONFIGURATION PARAMETERS

There are a number of fixed parameters which are compile time options

to the OS. These are identified near the beginning of the file OS.H,

with the title "General OS parameters" - Please notify me if you need any

of these parameters changed.

.p

> PANIC DEBUGGER

When an exception occurs, the operating system, all tasks and interrupts

are stopped. A 'PANIC' message indicating the cause of the exception is

output, If debugging is enabled, the PANIC debugger will be activated,

allowing you to debug the system.

>> Arm State Display

On entry to the panic debugger, the state of the ARM processor is displayed

with the 'cause' message:

.f-

Example:

1:	PANIC: Access violation at F0000000

2:	PC=000005C4 CPSR=80000050 Flags:Nzcvq-iFt Mode:USR

3:	USR SPSR=800000D0 SP=60009F1C LR=00006AD8

4:	SVC SPSR=00000050 SP=6000C000 LR=00004444

5:	IRQ SPSR=40000050 SP=6000B000 LR=000002A0

6:	FIQ SPSR=F00000FF R8=EFDFFFDF R9=FFFFFFFC R10=7B955FFF

7:	    R11=F7FFFFF7 R12=FFF9C3FE SP=FFFFFFFF  LR=BFE5BFFE

8:	R0=00000000  R1=00000000  R2=00000000  R3=00000020  R4=00000100

9:	R5=000065A8  R6=00006C2C  R7=60009F27  R8=F0000000  R9=00000000

10:	R10=00000000 R11=60009F98 R12=60009EFC

1    - Panic cause report

2    - Current program counter and status register

(with flag/mode interpretation)

3    - User mode banked registers

4    - Supervisor mode banked registers

5    - IRQ mode banked regsters

6-7  - FIQ mode banked registers (the values are

uninited because we don't currently use FIQ)

8-10 - General ARM registers

.f+

.p

>> Expressions

Anywhere that the debugger is expecting a value as a command parameter,

you can use the following:

.f-

x				= Hex value

.n				= Decimal value

reg-name		= Content of any register:

R0-R12	= general ARM register R0 to R12

PC		= current program counter (R15)

CPSR	= current processor status register

USPSR	= USR-mode Saved Processor Status Reg.

USP		= USR-mode Stack Pointer (R13).

ULR		= USR-mode Link Register (R14)

SSPSR	= SVC-mode Saved Processor Status Reg.

SSP		= SVC-mode Stack Pointer (R13).

SLR		= SVC-mode Link Register (R14)

ISPSR	= IRQ-mode Saved Processor Status Reg.

ISP		= IRQ-mode Stack Pointer (R13).

ILR		= IRQ-mode Link Register (R14)

FSPSR	= FIQ-mode Saved Processor Status Reg.

FSP		= FIQ-mode Stack Pointer (R13).

FLR		= FIQ-mode Link Register (R14)

FR8-FR12= FIQ-mode banked register R8-R12

-value			= Negation of value

~value			= Bitwise compliment of value

*value			= Word from memory at address [value]

@value			= Byte from memory at address [value]

.f+

Additionally, you can combine values with the following operators:

.f-

+				= Addition

-				= Subtraction

*				= Multiplication

/				= Division

%				= Modulus (remainder)

&				= Bitwise AND

|				= Bitwise OR

^				= Bitwise XOR

<<				= Shift left

>>				= Shift right

.f+

Evaluation of expressions is from right to left with no operator precedence.

You can force precedence with the use of round brackets:

.f-

2+3*4	= .20

2+(3*4)	= .14

.f+

.p

>>> Expression ambiguities

Some commands accept multiple parameter values, and this can lead to

ambiguities with the use of '-' and '*' as these can both indicate an

operator and a value.

For example, you might enter:

.c DUMP 1000 *R0

Intending to dump memory from address 00001000 to the address in the word

addressed by R0, however the expression parser would interpret this as:

.c DUMP (1000*R0)

And display the address in R0 multiplied by 1000 (hex).

In cases where a second (or third etc.) argument begins with '-' or '*',

you can avoid this ambiguity by enclosing the argument in brackets:

.c DUMP 1000 (*R0)

.p

>> Panic debug commands

The following commands are available within the panic debugger.

>>> BOOT

Restarts the system.

>>> BREAK [0-7] [address]

You can set up to 8 code breakpoints. On hitting a breakpoint, the

debugger will be entered with the PANIC message "Breakpoint n" where

'n' is the index number (0-7) of the breakpoint.

Setting an address of 0 will remove the breakpoint. Issuing 'BREAK' with

no operands will display the currently configured breakpoints.

Breakpoints are transparent - they are implanted in the code when running,

and removed when the debugger is active.

Breakpoints 1-7 are "sticky" they will remain in effect until removed.

Breakpoint 0 is reserved as a special "next time only" breakpoint which

disappears the next time the debugger activates.

NOTE: When CO