.T LCDtools

.=m5

.s15

.c ** LCD Tools **

.s

.c Tools for Building

.s

.c Mapping & Character Generation

.s

.c tables for custom

.s

.c Liquid Crystal Displays

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s10

.c ?COPY.TXT 2002-2005 Dave Dunfield

.c **See COPY.TXT**.

.P

> INTRODUCTION

Many modern embedded designs utilize custom liquid crystal display, which can

be quite complex. Due to this complexity and limitaions in the construction

of the displays, the mapping of display segments to hardware addresses in the

LCD controller rarely works out to anything which can be easily manipulated

from a software point of view.

For example, a display I recently worked with had this layout:

.f-

Top row   : 15 5x5 dot matrix characters

Middle row: 13 7-segment digits + one dash only digit

Bottom row: 12 7-segment digits + 11 misc words/symbols

.f+

The matrix design had to pretty much "square", which ment that each byte

in the LCD controller memory had a few bits from a dot-matrix character,

and a few assorted segments from the 7-segment and misc. areas. To make

matters worse, the mapping changed every time a misc. symbol occured,

so that the bits used in a dot-matrix or 7-segment character were not the

same from position to position. This made it impossible to directly address

the display from a character generator.

To write characters on a bit mapped display, you need to use a character

generator. This is simply a table which tells you which bits should be turned

on for a given character. To avoid having to have a separate character

generator table for each display position, you need to have exactly the same

relative bit mapping with each character of a given type. Then you can select

any character position on the display simply by adjusting the starting address

within display memory where you write the bits from the character generator

table. Clearly if the mapping within each character position is not identical

(such as in the display above), this technique will not work.

Unless you have a square dot-matrix display, most custom displays do not

have such nice "software friendly" mapping. The usual method of handling

this is to build a table which translates linear bit addresses which are

convienient for the software into the haphazard memory layout which suits

the hardware. This table should be organized so that the display layout

presented to the software is idealy suited for character generator output.

Building these tables "by hand" is a tedious process. LCD Tools simplifies

the job by allowing you to define the LCD layout visually on your PC screen,

and to define hardwre and software mapping simply by selecting segments with

the mouse. A character editor is also provided, providing easy on-screen

visual creation of character generator tables, with the resulting characters

displayed on the actual LCD image.

LCD Tools also provides an emulation mode, which allows you to use the PC

representation created above as a functional display on your target system.

This makes it easy to try out new LCD display designs before committing to

production, and also provides a way to continue with product software

development while waiting for the LCD panel to be manufactured.

.P

> USING LCD TOOLS

Tha main LCD Tools program is called "LCD". To start this program, simply

type LCD at the DOS prompt. LCD will start and present you with it's main

operating screen.

>> Main Screen Layout

The top 1/2 of this screen is a 320 x 102 dot-matrix representation of the

LCD panel. Within this screen, you can define up to 1024 LCD segments,

which can each consist of any combination of display "dots" from a single

dot, to the entire panel.

Immediately below the LCD display, is a text line which indicates status

and error messages which may be generated during the use of the program.

Below this is a line indicating the current display magnification, the

visible panel area (in rows - colums), and the current cursor position.

The next line displays the currently selected hardware physical segment

number (1-1024), or is blank if no segment is selected, indicating

unassigned panel area). This display also includes the offset into LCD

display memory in both hex-address and hex-address.bit forms (refer to

whichever is suited to your hardware design).

The next line displays the currently selected software mapped segment

number (1-1024), or is blank for no software segment mapping assigned.

This display also includes the hex offset into LCD bitmap table memory.

Below these lines is a text "help" display which shows the key functions

which can be performed.

>> Main Screen Functions

The main screen is primarily used for defining the LCD screen, assigning

the hardware segment mapping, and for activating other functions.

For most operations, the ENTER key will perform the same function as the

LEFT mouse button, and the ESC key will perform the same function as the

RIGHT mouse button.

>>> Mouse Functions

Pressing the LEFT mouse button will assign the display dot under the flashing

cursor to the hardware segment shown by the "Seg:" prompt. If no hardware

segment is selected (no "Seg:" visible), then the dot will be unassigned and

set to unused panel space.

Pressing the RIGHT mouse button will "pick up" the segment number under the

cursor, making it the current selection.

.p

>>> Keyboard Functions

.f-

Key(s)              Function

-----------------------------------------------------------------

LEFT,ENTER          Assign current hardware segment to display

RIGHT,ESC           Pickup hardware segment number from display

+,=                 Increases display magnification

-,_                 Decreases display magnification

Up/Down/Left/Right  Move display view (when >x1 magnification)

^Left               Adjust LCD display contrast

^Right              Adjust LCD display brightness

Ins                 Reset to full display view

Del                 Activate display "Zoom"

- Use mouse to position "window", press LEFT,ENTER and

display will "zoom" to that position.

- Use +,= and -,_ to adjust the size of the "window"

- Press RIGHT,ESC to exit without zoom.

PgUp/PgDn           Select hardware segment number (by 1)

^PgUp/^PgDn         Select hardware segment number (by 10)

Home/End            Select software mapping number (by 1)

^Home/^End          Select software mapping number (by 10)

F                   Block fill segment

- Select start  point, press LEFT

- Select ending point, press LEFT

* Entire block will be be assigned hardware segment number

C                   Copy block to clipboard

- Select start  point, press LEFT

- Select ending point, press LEFT

* Block will be copied to clipboard for later P)aste

* Clipboard is limited to 16384 dots (approx 1/2 display area)

M                   Software Mapping functions

- Press 'C' to clear the software map (no mapping)

- Press 'F' to find the segment with currently selected map

- Press 'H' to auto-assign 1-1 Software=Hardware mapping

* Used if hardware map is same as software map

* Also useful for creating "dummy" displays for emulation.

(Simply map hardware the way you want)

P                   Paste Clipboard

- Content of clipboard is displayed at mouse

- Position to desired location and press LEFT to copy the

clipboard block into the display at this point.

R                   Redraw screen (restore markers etc.)

S                   Set "Snap" spaceing

- Prompts with "Snap?"

- Enter value, cursor will "Snap" to this spacing.

* Value of 0 will turn Snap off.

L                   Draw Line

- Select start  position, press LEFT

- Select ending position, press LEFT

* Segments along line will be assigned hardware segment number

F1                  Set base marker

- Pick a point on LCD and press LEFT

* Cursor position will be shown relative to this marker

F2                  Assign hardware segments - Block

- Select start  position, press LEFT

- Select ending position, press LEFT

- All existing segments within the block will be reassigned

with the new hardware segment number.

- Repeat for as many blocks as you like

- Press SPACE to toggle On/Off auto-increment to next hardware

segment number with each block.

- Press RIGHT,ESC to exit

* Useful to re-assign complex segments after copy/paste.

F3                  Assign hardware segments - Shape

- Select a segment dot on the screen, press LEFT

- This and all adjacent dots (vertical and horizontal) with the

same hardware segment number will be reassigned with the new

hardware segment number.

- Repeat for as many shapes as you like.

- Press SPACE to toggle On/Off auto-increment to next hardware

segment number with each block.

- Press RIGHT,ESC to exit

* Useful to re-assign simpler segments after copy/paste.

F4                  Adjust hardware segments - Block

- Select start  position, press LEFT

- Select ending position, press RIGHT

- All segments within the block are reassigned beginning with

the new hardware segment number, keeping the relative segment

numbering of the original block.

- Press SPACE to toggle On/Off auto-increment to next unused

hardware segment number after this operation completes.

F5                  Assign software mapping number

- Position mouse over an assigned hardware segment, press LEFT

- Segment will be assigned displayed software mapping number.

- Repeat with as many segments as you like.

- Press SPACE to toggle On/Off auto-increment to next software

map number.

- Press RIGHT,ESC to exit.

F6                  Assign software mapping to block

- Select start  position, press LEFT

- Select ending position, press LEFT

- All segments within block will be assigned an ascending map

number (from current map number) beginning with upper left

corner, and ending with lower right corner, and scanning for

new segments row by row.

- Press SPACE to toggle On/Off auto-advance of map number on

exit.

F7                  Character generator editor

- Prompts with "Filename?", enter existing character generator

to use, or press RIGHT,ESC to begin a new one.

- Select upper left  corner of character block, press LEFT

- Select lower right corner of character block, press LEFT

- You are now in character editing mode:

- Click (LEFT) on a character segment to Toggle On/Off

- Press ASCII character to switch to a specific character

- Use Left/Right to switch character by 01

- Use Up/Down to switch charcter by 10 (hex)

- Use Ins for full screen view

- Use Del for zoom

- Use ^Left/^Right to adjust contrast/brightness

- Use RIGHT,ESC to activate menu

'S' = Save file

'Q' = Quit (no save)

F8                  Audit LCD layout

- Performs tests of the LCD layout and reports:

- Duplicated MAP addresses (error)

- Skipped MAP addresses (warning)

- Skipped Hardware Segments (informational)

F9                  Emulate LCD panel

- Prompts with COM=n BAUD=speed

- Use Up/Down to select COM port

- Use Left/Right to select baudrate

- Press ENTER when finished

- The display panel will be emulated via the serial data from

the target (See EMULATION COMMANDS)

- Use INS or DEL to clear screen

- Use ^Left/^Right to adjust Contrast/Brightness

- Press F1 to clear text area (under screen)

- Press ESC to exit

F10                 File menu

- Press 'L' to load an LCD description file

- Press 'S' to save current description to file

- Press 'Q' to exit program

.P

>> Steps to create an LCD layout

The following is the recommend steps for creating an LCD layout:

(Note, you should save your layout after each step)

>>> 1 - Define physical LCD segments

Using the main screen, define the visible segments using the LEFT mouse

button to assign single dots, or the F)ill and L)ine functions to create

larger shapes.

You may choose to assign the hardware segment numbering as you create the

segments, or to create them all with the same segment number (I use 1024),

and then reassign the hardware mapping afterward.

For displays with many identical elements (eg: 7-segment or dot-matrix

displays), it is usually best to create a single element, and then use

the C)opy and P)aste functions to copy the elements as many times as

you like.

>>> 2 - Assign hardware mapping

If you did not do so in step #1, you should now use the F2, F3 and F4

functions to assign the correct hardware mapping for each segment on the

display.

Note that if you have identical display elements which are mapped in

the same relative ordering, you can set the relative segment numbering

on the first element before you C)opy it to the clipboard, then after

you P)aste each new element, you can use the F4 function to automatically

assign the segment of the new block to a new range of hardware segment

numbers.

If you are mapping an existing display configuration, it may be useful to

write a small program on the target system which allows you to step an ON

segment through the display memory. This way you can simply step to each

new address, view the display segment, and click the corresponding segment

on the PC view to map that segment.

.p

>>> 3 - Assign software mapping

Once you have all display segments mapped to their corresponding hardware

address, you need to assign the software addresses. Keep in mind that the

software addresses should be laid out in a manner which allows character

generation. For example, if you have a display composed of several 5x5 dot

matrix characters, you might want the first character to be assigned software

addresses 1-25, the second 26-50, the third 51-75 etc.

In this example, you could calculate the starting address of any character

position by simply multiplying the character position by 25. As long as each

character position has the same relative mapping, all you need to do to write

a specific character on the display at that position is to copy the 25 bits

from the corresponding character generator table entry into the display memory

starting at that address.

You can use the F5 function to assign software addresses to individual

hardware segments, or the F6 function to automatically scan a block of

segments and automatically assign software addresses in a left-to-right,

top-to-bottom matrix.

>>> 4 - Verify the LCD Layout

Use the F8 function to audit the LCD layout.

- There should be NO duplicate MAP addresses. This is an error and

will result in a bad map table.

- There should be NO skipped map addresses. If this occurs, the map

table will still be correct, however it will contain zero entries

for the unused map locations, which wastes space, and would require

that your application driver be able to handle these special cases.

- There may or may not be skipped hardware segment addresses. Make

sure any segments reported are in fact NOT used on your display.

Be sure to correct any problems in the LCD layout before proceeding.

>>> 4 - Define character generators

Use the F7 functon to activate a character generator editor, select a

suitable display element (dot-matrix character for example), then simply

click on segments to turn them ON/OFF for each character position. (It

helps to use DEL-Zoom to view a large image of just that character

position).

Be sure to save your character generator before you exit.

Repeat this step for each type of display element for which you need a

character generator table.

>>> 5 - Build the Bit->Hardware map table

Use the LCM command to create an assembly language source file containing

the logical-bit to hardware-display-address table that your target will

need to address the LCD.

.p

>>> 6 - Build the Character Generator tables

Use the LCG command to create assembly language source files containing

the character generator Character-Code to Bit-Pattern tables that your

target system will need to translate character codes into displayed

characters.

>>> 7 - Write application drivers

Write the software drivers for your target application which will use

the above tables to provide simple and straightforward logical addressing

and character output to your LCD screen.

.P

> EMULATION COMMANDS

When operating in emulation mode (F9 command), the following single byte

commands are accepted from the target system via the serial line:

.f-

0xxxxxxx    - ASCII data

10000000    - Clear LCD screen

100xxxxx    - Report event flag (xxxxx=00001-11111 [1-31])

101xxxxx    - Set high segment address

110xxxxx    - Turn segment OFF

111xxxxx    - Turn segment ON

.f+

>> ASCII data

All ASCII data received from the serial port is displayed in the

text portion of the screen. NOTE: This portion does NOT scroll... When

the display advances to a new line, the screen wraps to the top of the

display area if it was previously on the bottom line, and the new

display line is cleared.

>> Clear LCD screen

This command causes all segments of the emulated LCD panel to be

turned OFF.

>> Report event flag

Displays the string "[n]" (n=1-31) in the text window using the color

RED. This provides a simple 1-byte way of reporting up to 31 different

software events.

>> Set high segment address

This command sets the upper 5 bits of the 10 bit segment address

(segments are numbered from 0 to 1023). This command does not have to

be sent for every segment update. It need only be sent when the upper

5 bits of the segment number to be updated has changed.

>> Turn segment OFF

Use the lower 5 bits included in this command and the higher 5 bits

sent previously to determine a 10 bit segment number (0-1023) and turns

that logical segment OFF (Note: The segment number refers to the software

mapping number assigned by the LCD editor).

>> Turn segment ON

Use the lower 5 bits included in this command and the higher 5 bits

sent previously to determine a 10 bit segment number (0-1023) and turns

that logical segment ON (Note: The segment number refers to the software

mapping number assigned by the LCD editor).

.P

> LCM - MAP TABLE BUILD UTILITY

The LCM utility is used to build the software -> hardware map table which

is used by your application driver to address the LCD panel in a logical

way.

The output of LCM is assembly language source code, which is written to

the standard output device. You can redirect this output to a file with

the '>' command line operator:

.c eg: LCM lcd_file [options] >output_file

'lcd-file' is the name of the .LCD file created using the LCD utility.

It contains the complete segment/mapping definitions for your LCD.

'output_file' is the name of a new file to receive the LCD mapping table

in the form of assembly source code.

>> Command Line Options

.f-

-I        = "Intel format" when this option is specified, hex

byte values are written to the output file in the

form: 0xxh where 'xx' is the hex value. When this

option is NOT specified, output values are in the

form '$xx'.

-L        = "Little endian" when this option is used, 16 bit

values are reversed before output. This may be

needed for systems which expect word values to

occur low byte first.

-R        = "Reverse bit masks". For the output formats which

contain a bit mask, this reverses the mask, so that

bit 7 becomes bit 0 and vice versa.

C=text    = "opening Comment". This option causes LCM to insert

a comment containing the specified text at the start

of the output file. Note that the '_' character will

be translated to a space.

D=text    = "Define directive". Sets the assembly language

directive that LCM will use to define bytes/words.

F=n       = "output Format". Specifies the type of output data that

will be generated for each table entry:

1 = single byte address     :       DB  $12

2 = byte address.bit-mask8  :       DB  $53,$40

3 = word address.bit-mask8  :       DB  $01,$36,$20

4 = word address            :       DW  $0157

5 = word address.bit-mask16 :       DW  $0115,$8000

L=n       = "operand Length". Specifies the length (in characters)

of each output operand at which LCM will begin a new line.

N=text    = "Name". Causes LCM to output a symbolic label containing

this name at the beginning of the table.

.p

O=hex     = "Offset". Specify an offset (in hex) which LCM will add

to each table address before output.

S=hex     = "Step". Specify the distance (in hex memory locations)

between each address within the hardware LCD memory.

.f+

LCM will generate one table entry for each logical (map) address defined

within the LCD module. This entry will contain the physical hardware address

for the corresponding segment in the selected format. The most commonly

used format is an address and bit mask. The address represents the memory

location/register within the LCD controller, and the bit mask represents

the individual bit which must be set/cleared to turn ON/OFF the segment.

>> Example Table Processing

For example, when the software wishes to turn ON logical segment number

123, it would look in entry number 123 of the mapping table, where it

might find: $57,$04. These entries indicate that bit 2 (mask=04) must

be set in location $57 to turn on this segment.

Example RTS8505 code (6502 core) to do this, given that the memory

location LCDADR contains the logical bit address within this table,

the following code might be used to turn on the corresponding segment:

.f-

LDY     #0                  ; Zero offset

LDA     [LCDADR],Y          ; Read LCD hardware address

TAX                         ; X = LCD address

INY                         ; Advance to mask

LDA     [LCDADR],Y          ; Read the bit mask

ORA     0,X                 ; Set this bit

STA     0,X                 ; Set new value

.f+

To clear the bit, you might use:

.f-

LDY     #0                  ; Zero offset

LDA     [LCDADR],Y          ; Get LCD hardware address

TAX                         ; X = LCD address

INY                         ; Advance to mask

LDA     [LCDADR],Y          ; Read the masl

EOR     #%11111111          ; Invert for AND

AND     0,X                 ; Remove the bit

STA     0,X                 ; Set new value

.f+

.P

> LCG - CHARACTER GENERATOR BUILD UTILITY

The LCG utility is used to build the character generator bitmap table(s)

which are used by the application to write arbitrary characters onto the

LCD screen through the logical mapping table.

The output of LCG is assembly language source code, which is written to

the standard output device. You can redirect this output to a file with

the '>' command line operator:

.c eg: LCG lcg_file [options] >output_file

'lcg-file' is the name of the .LCG file created using the LCD utility,

containing a character generator definition.

'output_file' is the name of a new file to receive the character generator

bitmap table in the form of assembly source code.

>> Command Line Options

.f-

-I        = "Intel format" when this option is specified, hex

byte values are written to the output file in the

form: 0xxh where 'xx' is the hex value. When this

option is NOT specified, output values are in the

form '$xx'.

-R        = "Reverse bit pattern". Causes LCG to write the bit

pattern with the first bit in bit-0 of each byte.

If this option is NOT specified, LCG writes the first

bit in bit 7 of each byte.

C=text    = "opening Comment". This option causes LCG to insert

a comment containing the specified text at the start

of the output file. Note that the '_' character will

be translated to a space.

D=text    = "Define directive". Sets the assembly language

directive that LCM will use to define bytes.

L=hex     = "Low bound character". Specifies the first character

code for which LCG will begin generating table entries.

If not specified, the lowest character position defined

in the character generator is used.

H=hex     = "High bound character". Specifies the last character

code for which LCG will geneate table entries. If not

specified, the highest character position defined in

the character generator is used.

N=text    = "Name". Causes LCG to output a symbolic label containing

this name at the beginning of the table.

.f+

The tables produced by LCG contain the bit pattern which must be written

to the logical LCD segments to produce the corresponding character.

.p

>> Example Character Processing

Lets assume:

.i+2

.i-2 - An LCD mapped with a 10 character 5x5 dot matrix display beginning

at logical segment 98. Each 5x5 character consists of 25 dots, which are

mapped such that the first digit is dots 98-122, second digit is dots

123-147, the third is dots 148-172 an so on.

.i-2 - A 5x5 dot matrix character generator is defined. This table spans

the characters from 'space' ($20) to 'Z' ($5A).

.i-2 - We would like to display the character 'A' in the 3rd dot matrix

location.

.i-2

First, we need to compute the logical bit address for the 3rd dot matrix

character. To do this, we multiply the number of digits occuring BEFORE

the character we desire by the size of each character in bits, and add

the offset to the start of the dot-matrix display:

.f-

2          <= Characters before 3rd

x   25          <= Dots / character

------

=   50          <= Total # of dots before this character

+   98          <= Beginning address of dot matrix

------

=  148          <= Beginning address of 3rd character

.f+

Then, we need to find the character generator table entry for the

character 'A'. To do this, we take the character code ('A'=$41) and

subtract the first character in the table (in this case, our table

starts at $20). The result must be multipled by the size of each

table entry, which in this case is 4 bytes (25 bits needs 4 bytes):

.f-

$41          <= 'A' code

-  $20          <= First code in table (space)

------

=  $21          <= # of characters into table

*    4          <= # of bytes/entry

------

$84          <= Offset into table where 'A' starts

.f+

Then, to draw the 'A' on the LCD screen at the 3rd dot matrix character

position, we simply read the 25 bits at offset $84 in the character

generator table, and write those bits through the logical mapping table

beginning at dot position 148 ($94 hex)... Voila! the character 'A' will

appear at the 3rd dot matrix character position.

.P

> SAMPLE FILES

The following sample files are included with this package:

>> PANEL1.LCD

This is the layout of the actual 562 segment LCD for which I originally

created LCD tools. This was attached to a RealTek RTS8505 Caller ID using

exactly the segment mapping shown. Note the haphazard hardware addressing,

including many segments which are simply "skipped" in the hardware map.

>> PANEL2.LCD

This is a prototype LCD panel for a feature telephone, which incorporates

dot matrix, 7 segment numeric and 16 segment alphanumeric displays. Since

this is a prototype, no physical display exists (yet), and no hardware

mapping is available. To easily map this display, I simply created the

segments in "logical" order and used the M)ap H)ardware function to map

the display 1-1.

When the actual LCD gets laid out and I have the hardware map information,

I will simply re-address and re-map the display using LCD Tools.

>> 25DOT.LCG

This is a 25 dot (5x5) dot-matrix character generator, which spans ASCII

codes from $20 (space) to $60 (`). It can be used on either PANEL1 or

PANEL2.

>> 7SEG.LCG

This is a 7 segment character generator which spans ASCII codes from $20

to $60 (`). Although not all of these codes will be used (some letters are

a bit odd), having the same character generator size for all three display

areas in PANEL2 allow me to have a single character output function which

selects the appropriate character generator table based on the logical

display address. This character generator can be used on either PANEL1 or

PANEL2.

>> 16SEG.LCG

This is a 16 segment character generator which also spans ASCII codes from

$20 to $60 (`). It is applicable to PANEL2 only, as PANEL1 does not have

any 16 segment alphanumeric display elements.