.T MICRO-Terminal

.=m5

.s20

.C MICRO-Terminal:

.s

.c A tiny Terminal

.c program for the

.s

.c IBM/PC

.s

.c with XMODEM

.s5

.c Release 1.2

.s

.c Revised 05-Jan-98

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1989-2005 Dave Dunfield

.c **See COPY.TXT**

.P

> INTRODUCTION

MICRO-Terminal is a tiny terminal program, which supports a subset of

the ANSI control codes (Also known as VT100 control codes), and basic XMODEM

binary file transfer. The MTERM.COM program file occupies only 10K of disk

space, which makes it an ideal program for use on a laptop or other floppy

disk based system.

It is also very useful for communications debugging, since it loads fast,

is easy to setup, supports all asynchronous data formats, and has a TSR

(POP-UP) mode which enables it to be kept in memory and activated with a

simple keypress combination.

NOTE: MICRO-Terminal is a SUBSET of the ANSI specification, and

will not pass all of the "VT100 - Checkup" tests.

> Using MICRO-Terminal

>> Normal mode

The MICRO-Terminal program may be used in two ways, the first is as a

"normal" DOS program. To activate MICRO-Terminal in this way, enter the

command:

.c MTERM

This will load MICRO-Terminal, and present you with the main function

menu, allowing you to access its features. When you select the "Exit to DOS"

option, MICRO-Terminal will terminate in the "normal" fashion, and return

you to the DOS command prompt.

>> POP-UP mode

The second way to use MICRO-Terminal is as a TSR (POP-UP) program. To

use this feature, you must specify HOTKEYS on the command line. Available

HOTKEYS are:

.f-

L - Left SHIFT

R - Right SHIFT

A - ALT

C - CONTROL

S - SysRq (Caution: some systems may not like this one)

.f+

For example:

.c MTERM LR

This will load MICRO-Terminal into memory, but will not activate it. You

will immediately be returned to the DOS command prompt. Once MICRO-Terminal

has been installed in this way, you can "POP" into it simply by pressing the

LEFT and RIGHT SHIFT keys at the same time. This allows MICRO-Terminal to be

accessed from ANYWHERE (even if you are running another program). When you

select the "Exit to DOS" function from the main menu, you will be returned

to whatever application program was running when you "POPPED" in.

.P

>> The main menu

When you enter MICRO-Terminal, it first presents you with a main menu,

which allows you to access its features, through the following selections:

>>> Terminal Emulation

This selection causes MICRO-Terminal to begin emulating the ANSI

terminal, allowing you to send and receive data to and from the COMM

port. Press CTRL-Home or CTRL-End to exit Terminal Emulation.

>>> XMODEM Download

When you select this option, you will be prompted for a filename. Once

the filename is entered, MICRO-Terminal will begin receiving data into that

file from the COMM port using the XMODEM protocol. A status window is

displayed showing the filename being written, the number of XMODEM 128 byte

blocks received, and any error conditions which occur during the download.

You may press the <ESCAPE> key at any time to abort the download, and

return to the main menu. NOTE: It make take a few seconds from when you

press <ESCAPE> until the download aborts.

>>> XMODEM Upload

When you select this option, you will be prompted for a filename. Once

the filename is entered, MICRO-Terminal will begin transmitting the contents

of that file to the COMM port using the XMODEM protocol. A status window is

displayed showing the filename being transmitted, the number of XMODEM 128

byte blocks sent, and any error conditions which occur during the upload.

You may press the <ESCAPE> key at any time to abort the upload, and

return to the main menu. NOTE: It make take a few seconds from when you

press <ESCAPE> until the upload aborts.

>>> Serial port config

This selection presents you with a sub-menu which allows you to

configure the COMM port, baudrate, number of data bits, parity type, number

of stop bits and XON/XOFF flow control to be used for the terminal session.

>>> Exit to DOS

This selection terminates the main menu, and exits the MICRO-Terminal

program.

.P

> Keypad keys

MICRO-Terminal maps the ANSI auxiliary keypad codes into the

IBM PC keyboard using the following table:

.f-

ANSI Key        IBM/PC Key      Code Transmitted

------------------------------------------------

0              F10              <ESC>Op

1               F1              <ESC>Oq

2               F2              <ESC>Or

3               F3              <ESC>Os

4               F4              <ESC>Ot

5               F5              <ESC>Ou

6               F6              <ESC>Ov

7               F7              <ESC>Ow

8               F8              <ESC>Ox

9               F9              <ESC>Oy

-           Keypad '-'          <ESC>Om

,           CTRL-PgUp           <ESC>Ol

.           CTRL-PgDn           <ESC>On

ENTER         Keypad '+'          <ESC>OM

PF1             Home             <ESC>OP

PF2             End              <ESC>OQ

PF3             PgUp             <ESC>OR

PF4             PgDn             <ESC>OS

.f+

> Control codes

MICRO-Terminal recognizes the following subset of the ANSI

terminal command codes:

.f-

Position cursor:        <ESC>[n;nH

<ESC>[n;nf

Clear to end of screen: <ESC>[J

"   "   "   "       <ESC>[0J

Clear entire screen:    <ESC>[2J

Clear to end of line:   <ESC>[K

"   "   "   "       <ESC>[0K

Clear entire line:      <ESC>[2K

Select normal video:    <ESC>[0m

* Select underline:       <ESC>[4m

Select reverse video:   <ESC>[7m

* This works on monochrome displays only.

.f+