.T PC Logic Analyzer

.=m4

.s20

.C PCLA

.S

.c An IBM P.C. Based

.C Logic Analyzer

.s5

.c Release 1.0

.s

.c Revised 27-Mar-00

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 2000-2005 Dave Dunfield

.c **See COPY.TXT**

.P

> INTRODUCTION

PCLA is a simple PC based Logic Analyzer which uses the parallel port

as an input device to monitor and record the activity of up to 13 TTL level

logic signals in real time. PCLA uses a standard bidirectional PC/LPT port,

and no additional hardware is required (you may want to provide some

protection for the parallel port as discussed later in this document).

PCLA is **NOT** a high-speed logic analyzer. The speed at which the PC

can poll the parallel port is limited, making PCLA a low speed device. For

most PC's of 486/33 or faster, PCLA will have a maximum sampling rate of

between 375 and 500 Khz (2.6-2.0us/sample). For faster PC's the speed is

limited by the ability of the chipset to perform I/O to the LPT port more

than it is by CPU speed.

>> Hardware Requirements

PCLA requires a 486/33 of faster PC/AT compatible running DOS 3.0 or

higher. A slower PC will work, however you may experience a reduced sampling

rate.

PCLA also requires a bidirectional parallel port. Most ports on modern

PC's have bidirectional capability, however some older ports are not. If the

older port is on a ISA card and is constructed from discrete TTL logic, you

may be able to make it into a bidirectional port with little difficulty (See

Appendix A). A non-bidirectional port can be used, however you will be

limited to monitoring the 5 control lines (A-E).

Finally, you will need a cable to connect the parallel port inputs to

your target system. See the following section for more information and ideas.

.P

> INTERFACING TO THE TARGET SYSTEM

In the simplest form, you could simply make a cable which connects the

required parallel port signals to the test points on the target system that

you wish to monitor, however there are some issues that you should be aware

of:

>> Connection Issues

The signals transmitted and received on the PC parallel port are TTL

level signals. These have a range of 0v (logic 0/system ground) to +5V

(logic high). A logic low should be less than 0.8v, while a logic high

should be greater than 2.4v. Connection of the parallel port signals to

any voltages outside of this range will damage the parallel port.

Pins 2-9 are the parallel port DATA pins, which are normally configured

by the PC as OUTPUTS. This means that if you connect these pins to your target

system, they will be sending power into the target until you run PCLA. This

has two major problems, 1) it may cause undesired operation of the target, and

2) signals from the target will be "fighting" with the signals from the PC,

which may stress the parallel port and/or target hardware and cause it to fail.

For this reason, it is strongly advised that you do not connect the

target system to the PC parallel port after turning the PC on, until you

have run PCLA at least once.

>> Pin Connections

PCLA reads it's data from the following pins of the parallel port:

.f-

Pin Signal  Pin Signal

-----------------------------

2 -Data 0  15 -Data A

3 -Data 1  13 -Data B

4 -Data 2  12 -Data C

5 -Data 3  10 -Data D

6 -Data 4  11 -Data E

7 -Data 5

8 -Data 6

9 -Data 7   18-25 -Ground

.f+

.P

>> Protective Circuitry

You should also be mindful of the possibility of damage to the PC parallel

port. As noted above, the PC port can be damaged by excessive stress from

conflicting outputs, however it can also be damaged by static discharge, or

by accidental contact with non-TTL level voltages in the target system. For

this reason, you may want to consider adding some protective circuitry to all

of the signals used on the parallel port.

The most basic protection circuit is a current limiting resistor. I have

found that a 150 Ohm resistor in series with each line works well to prevent

excessive current, while still offering a low enough resistance that TTL low's

will be correctly sensed.

Further protection can be achieved by using Diodes on the PC side of the

current limiting resistor to insure that the input voltage does not go above

+5v or below 0v (Ground). Since a 5v reference is not available on the PC

parallel port, you may wish to use a Zener diode to provide one. The drawing

file PCINPUT.DWG contains a MICROCAD format drawing of some example input

protection circuitry.

It is also worth mentioning that some parallel ports are much more

difficult to replace than others. If possible, I recommend using a cheap

parallel port Add-in card. If you damage it, you can simply replace it with

another one. If on the other hand, you damage the parallel port on your

mainboard, or worse, in your laptop computer, this can be very expensive

to fix!.

My own setup is an add-on LPT port with 150 Ohm series resistors,

which I have found to be quite satisfactory.

>> Physical Considerations

On the matter of physically connecting to the target system, I recommend

building a small circuit board which contains a DB-25 connector for connection

to the PC via an "off the shelf" cable, a couple of 150 Ohm "resistor packs"

which will provide 14 series resistors (you need 13) in two neat DIP packages,

and a couple of DIP headers for connection to the target system.

I arranged the DIP headers in two groups, one of 8 pairs for the 0-7

data lines, and one of 5 pairs for the A-E data lines. Ground all pins on one

side of the header, and connect the pins on the other side through the

resistor packs to the DB-25 connector. A standard ribbon cable and connector

will fit the headers and provide alternating ground and signal wires.

I used about a 1-foot long ribbon cable, and split it apart about halfway

along it's length (At this point, I just cut the ground wires off). The

remaining data signals should be connected to miniture "test point" clips,

the kind that have a small hook that can grab onto a test point pin or

device lead.

.P

> RUNNING PCLA

PCLA is a very timing intensive program, and for this reason it is

not recommended that you run it from within windows. Windows does not

provide the real-time processing ability which PCLA requires for accurate

data sampling.

If you are using a Windows9x system, you should use the "Restart

computer in MSDOS mode" option from the SHUTDOWN menu before running PCLA.

If you are using Windows 3.1, you should exit back to the DOS prompt

before running PCLA.

PCLA will not run under WindowsNT due to it's requirement for direct

access to the parallel port hardware (which WinNT does not allow).

It should also be noted that TSR's, device drivers and other resident

programs may perform background processing which will interfere with PCLA's

sampling. If you have such things loaded, you should unload them before using

PCLA. It may be desirable to create a bootable floppy disk which will enable

you to "clean boot" the system before running PCLA. See APPENDIX C for more

information.

>> Command Line Options

PCLA supports the following command line options.

.s2

.i+

.i- /D - Prevent unused entry detection in preview

If there are less then 256 preview entries, PCLA tries to identify unused

ones by using unstandardized features of the parallel port hardware. If you

are getting incorrect preview displays, use this option to disable this. Unused

entries will then show simply as all lines at logic 0.

.i- /O - Revert parallel data lines to outputs on exit

Normally, PCLA will configure the parallel port data lines to be inputs,

and will leave the parallel port configured this way when it exits. This is

generally the most convenient, as it prevents any signal conflicts from occurring

on these lines once PCLA has been run once.

If however you need to have the parallel port data lines revert to being

outputs after PCLA exits, you may use the '/O' option to accomplish this.

.P

.i- /O* - Revert parallel data lines to outputs and exit immediately.

This option causes PCLA to configure the parallel data lines as outputs,

and terminate immediately. No screen output is produced. This option is

most useful from within "batch" files to re-configure the parallel port

after a PCLA session.

.i- /I - Disable interrupts during capture

When you are capturing data at very high rates, the time taken to service

interrupts on the PC can cause sampling gaps. This occurs because the PCLA

code is polling the parallel port in a very tight loop, and has to "take time

out" to service the interrupt. The effect will be more pronounced on slower

machines.

The '/I' option disables all interrupts during capture except for the

keyboard interrupt (which is required in order to abort the capture). This

will prevent the sampling gaps, however it also means that the PC time-of-day

clock will not advance while PCLA is waiting for a trigger, or acquiring

data. This has the effect that the PC's clock loses time. To compensate

for this, PCLA will automatically set the PC time-of-day clock to the

time currently indicated by the PC's real time (CMOS) clock chip at the

end of every capture cycle when this option is used.

.i- /I* - Disable interrupts during capture but do not reset clock

This option causes PCLA to disable interrupts during capture as

described above, but prevents it from resetting the time-of-day clock.

This should be used if your PC has a non-standard clock chip, or you

do not want the clock reset for any other reason.

.i- /M - Force monochrome screens

PCLA will automatically detect a color adapter, and will use more

pleasing color screens when one is found. If you happen to be using a

monochrome monitor with a color adapter, you may need to specify the

'/M' option to force PCLA to use the monochrome screen format.

.i- /W - Display wiring information

This option causes PCLA to display a small summary of the parallel

port signal pins, and then terminate.

.i- I=n - Set default capture interval (microseconds)

This option sets the capture interval to the specified number of

microseconds. The capture interval defines the length of time which

PCLA waits between each sampling of the input data lines during the

"T"rigger and "A"cquire phases of data capture.

.P

.i- M=n - Set default monitor update interval (milliseconds)

This option defines the amount of time occurring between each sample

in the "monitor" screen. This time is derived from the PC clock tick,

and has a resolution of 55 milliseconds only.

.i- P=1-3/address - Set parallel port number/address

This option specifies the parallel port which will be used by PCLA.

A value of 1-3 indicates the port LPTn as detected by the BIOS. A value >3

indicates an absolute I/O address (in hexidecimal). The standard parallel

port addresses are: 3BC, 378 and 278.

.i- S=n - Set number of samples

This options sets the default number of samples which will be

collected during the "A"cquire phase of data capture.

.i-

> USING PCLA

When launched, PCLA starts up in "monitor" mode, where it polls the

parallel port at regular intervals, and draws a scrolling graph showing

the logic levels. This is a low-speed display, which is mainly intended

to let you see the current state of the logic inputs "at a glance".

>> Monitor mode

This "monitor" mode may also be considered the main screen, with all

functions of PCLA being accessed from subfunctions activated here:

.i+

.i-2 C)apture - Begin high-speed data capture

This function causes PCLA to begin data acquisition. The parallel inputs

are read at the sampling interval [See S)etup function], and compared with

the trigger mask [See T)rigger function]. This will be repeated until a match

occurs (note, if all triggers are set to "X - Don't care", the match will

occur immediately). The fact that PCLA is waiting for a trigger match is

indicated by "[T]" in the upper left corner of the display.

Once a trigger match occurs, PCLA will begin acquiring data. The parallel

inputs are read at the sampling interval and stored in memory. This will be

repeated until the defined number of samples has been stored [See S)etup

function]. When PCLA enters this "acquire" phase of the capture process, the

"[T]" in the upper left corner of the screen will change to "[A]".

At any time, you may terminate the capture process by pressing the

ESCAPE key.

.P

Once the capture process has completed, you may view the stored data

with the "Review" function. The 255 samples occurring immediately before the

trigger match may be viewed with the "Preview" function.

Note that if the "/I" option is in effect, the PC interrupts will be

disabled during the capture process (Both [T]rigger and [A]cquision). This

causes the PC time-of-day clock to lose time. In this case, PCLA will reset

the time-of-day clock to the real-time (CMOS) clock unless the "/I*" option

was given, in which case the time-of-day clock will lose time.

.i- F)ile - Perform File operations

This function allows you to load or store the captured data and certain

configuration parameters to and from a flie. You will be prompted for the

operation (Load or Save), and the filename.

The information saved and loaded from the file is:

.f-

- The configured sampling interval

- The configured number of samples

- The configured trigger information

- The position in the preview buffer

- The preview data

- The size of the review data

- The review data

.f+

.i- M)onitor - Set the monitoring interval rate

Prompts you with the current monitoring interval rate, and allows you

to change it. The monitoring interval rate is specified in milliseconds,

by is actually measured in "chunks" of 55 ms (the PC clock tick interval).

The monitoring interval rate defines how frequently PCLA will sample

the input lines and update the monitor display when operating in monitor

mode.

.i- P)review - Preview data before trigger of last capture

Enters the preview viewer, and allows you to view the 255 samples

which occurred immediately before the trigger match occurred on your last

C)apture function.

.i- R)eview - Review acquired data

Enter the review viewer, and allows you to view the logic data that

was acquired following the trigger match on your last C)apture function.

.P

.i- S)etup - Setup capture process

Prompts with the number of samples and sampling interval, and allows

you to change them. The new values will be in effect for all subsequent

C)apture functions.

"Number of samples" defines how many samples should be acquired and

stored following a trigger match before the capture function is over. A

value of zero is a special case, and indicates 65536 samples, the highest

number that PCLA can store.

"Sampling Interval" defines the period of time (us) which elapses

between each sample taken.

The total time of the acquisition will be the "Number of samples"

multiplied by the "Sampling Interval".

.i- T)rigger - Set capture trigger

Brings up a menu of the capture trigger. This has a setting for

each input line, which can assume one of three states:

.f-

0   - Bit has to be Zero to match trigger

1   - Bit has to be One  to match trigger

X   - Don't care - either 0 or 1 will match

.f+

You may press the letter ('0-7' or 'A-E') of an entry to toggle it

through the three possible states. Press ENTER when you are done.

When you activate the C)apture function, PCLA will wait until all

of the input lines match their respective trigger settings before beginning

to acquire and store the data.

.i- F10:Exit - Terminate PCLA

This causes PCLA to terminate, and return to the DOS prompt.

If the '/O' option was specified when PCLA was started, the LPT

port data lines are reverted to outputs.

.i-

.P

>> Reviewing Data

When you activate the R)eview function from monitor mode, PCLA

enters the acquired data viewer. This viewer shows the acquired data

on the screen in a format similar to monitor mode, and you may scroll

through the data with the following keys:

.f-

Left and Right arrows   - Move back and forth by 1 character

Up and Down arrows      - Move back and forth by 10 characters

PgUp and PgDN           - Move back and forth by 75 characters

Home                    - Position to the start of the data

End                     - Position to the end of the data

.f+

A marker line between the A-E and 0-7 displays shows the position of

the screen data within the data buffer. At the left end of this marker

line is a number representing the position of the leftmost visible data

entry. At the rightmost end of the marker line is a number representing

the position of the rightmost visible data entry (Note: 0 is a special

case, and means 65535 - the highest entry number which can be recorded).

Along the marker lines, each position is identified by a specific

character, as follows:

Positions evenly divisible by 10 are marked with a square.

Positions evenly divisible by 5 are marked with a large circle.

All other positions are marked with a small circle (dot).

In addition to moving around and viewing the acquired data, the following

functions are available in review mode:

.i+

.i- F1:Search - Search for logic pattern

Prompts with a match qualifier, which contains an entry for each

input line which can assume one of three states:

.f-

0   - Bit has to be Zero to match

1   - Bit has to be One  to match

X   - Don't care - either 0 or 1 will match

.f+

You may press the letter ('0-7' or 'A-E') of an entry to toggle it

through the three possible states. Press ENTER when you are done.

PCLA will then search forward through the buffer, beginning at the

current display position, until it finds an entry in which all of the

input lines correspond to the entries defined in the match qualifier.

If such an entry is found, the viewing position is moved to that entry.

.P

.i- F2:Search Again - Search for next occurance of logic pattern

This function begins searching for a logic pattern, using the

match qualifier from the previous F1:Search command, and beginning

at the current display position in the buffer +1. This effectivly

searches for the next occurance of the logic pattern.

.i- F3:Count - Count state changes

Prompts with a state qualifier, which contains an entry for each

input line which can assume one of two states:

.f-

X - Don't care  - Ignore changes in this line

* - Do care     - Notice changes in this line

.f+

You may press the letter ('0-7' or 'A-E') of an entry to toggle it

through the three possible states. Press ENTER when you are done.

PCLA will then scan from the current position to the end of the buffer,

and count how many times the state of all '*' lines changes (A change on

one or more '*' line constitutes a state change). The count of state changes

is then displayed.

To determine the state changes within a section of the buffer which does

not reside at the end of the buffer, perform this function at the start of

the section, and again at the end of the section and subtract the values.

.i-

>> Previewing Data

When you activate the P)review function from monitor mode, PCLA

enters the preview data viewer. This viewer shows the last 255 samples

which were taken immediately before a trigger match was made. You may

scroll through the data with the following keys:

.f-

Left and Right arrows   - Move back and forth by 1 character

Up and Down arrows      - Move back and forth by 10 characters

Home                    - Position to the start of the data

End                     - Position to the end of the data

.f+

.P

A marker line between the A-E and 0-7 displays shows the position of

the screen data within the data buffer. At the left end of this marker

line is a number representing the position of the leftmost visible data

entry. At the rightmost end of the marker line is a number representing

the position of the rightmost visible data entry.

Along the marker lines, each position is identified by a specific

character, as follows:

Positions evenly divisible by 10 are marked with a square.

Positions evenly divisible by 5 are marked with a large circle.

All other positions are marked with a small circle (dot).

NOTE: Positions shown in preview mode indicate the sample number BEFORE

the trigger match, and therefore get larger as you move chronologically

backward in time. Position 0 at the end of the buffer indicates the input

lines at the moment of trigger match.

ALSO NOTE, PCLA does not keep track of the beginning of the circular preview

data buffer. It does keep track where the end is located, and will display

the last 256 entries in order from the End back. If the trigger match

condition occurs LESS THAN 256 entries after the capture begins, the preview

buffer will not all contain valid data. PCLA tries to identify unused entries

by using unstandardized features of the data read from the port, however this

may not work correctly on all PC's. If you are getting no-data indicators in

the middle of your preview data, try using the '/D' option to disable this

detection. In this case, unused preview entries will show all lines at logic

zero (0).

In addition to moving around and viewing the acquired data, the following

functions are available in review mode:

.i+

.i- F1:Search - Search for logic pattern

Prompts with a match qualifier, which contains an entry for each

input line which can assume one of three states:

.f-

0   - Bit has to be Zero to match

1   - Bit has to be One  to match

X   - Don't care - either 0 or 1 will match

.f+

You may press the letter ('0-7' or 'A-E') of an entry to toggle it

through the three possible states. Press ENTER when you are done.

PCLA will then search backward through the buffer, beginning at the

current display position, until it finds an entry in which all of the

input lines correspond to the entries defined in the match qualifier.

If such an entry is found, the viewing position is moved to that entry.

.P

.i- F2:Search Again - Search for next occurance of logic pattern

This function begins searching for a logic pattern, using the

match qualifier from the previous F1:Search command, and beginning

at the current display position in the buffer -1. This effectivly

searches for the next occurance of the logic pattern.

.i-

> OPERATIONAL TIPS

>> Aliasing

When a signal is being sampled, state changes which occur on the signal

after one sample, but before the next will not be visible until that sample

has occurred. This causes an effect called aliasing, in which the waveform

that you see is not exactly the same as the one being sampled if the sample

rate is close to the state change rate.

Consider a square wave changing at a rate of 3us per state change.

If we sample this at 2us and at 4us,  we will see the following:

.f-

___---___---___---___---___---___---___ Original Signal

0..3..3..3..3..3..3..3..3..3..3..3..3..

___---____--____--____--____--____--___ 2us sample

0.2.2.2.2.2.2.2.2.2.2.2.2.2.2.2.2.2.2.2

____----________----________----_______ 4us sample

0...4...4...4...4...4...4...4...4...4..

.f+

Note that when we sampled slightly faster than the state change

rate, we saw distortion of the waveform, however we did see all of

the state changes. This type of aliasing is usually not to detrimental

to the non-timing critical logic analysis that PCLA is normally used

for...

But! Note what happened when we sampled at a rate slower than the

signal state changes... We completely lost some state changes, and in

fact, this case happens to present the signal as 1/2 the frequency of

the original! This is not good, and clearly shows why you need to make

sure that you are sampling your signals at least as fast as they can

change state.

Normally, aliasing won't be so regular, but will occur only at points

where the target system clock and PC sampling rates are in a specific

relationship. This relationship will come and go as the two unsynchronized

devices drift. Be warned that if you are sampling at rates below the

state change period of the target system, what you are seeing may be far

from the truth!

.P

>> Slowing it down

PCLA samples at rates up to 500khz on some computers, however most

small microccontrollers run at several megahertz, and often higher...

If you are trying to resolve some hardware timing issue that is

related to "race conditions" and other speed related issues, then forget

PCLA. You need a fast hardware logic analyzer and a lot of patience!

If however you are trying to debug a straightforward clocking or

synchronization issue, PCLA can be very handy, even if the target device

is running faster than PCLA can sample.

>>> Software Solutions

If the signals you are monitoring are being generated by software,

such as "bit banging" a clocked serial line for a serially accessed

peripheral, then the solution is simple... Slow down the software!

This can usually be accomplished by putting delay loops into the code

at the points where it is toggleing the I/O lines.

Once you get the process running slowly enough for PCLA to capture

it, you can verify the logic, make fixes, and once you get it working

at the slower speed, just remove your loops, and watch it work at high

speed! (if not, the problem should be fairly easy to track down as it

is obviously timing related).

>>> Hardware solutions

If the signal you are monitoring is generated by the hardware (such

as a microprocessor bus), or if it is impractical to slow down the software

by code changes (perhaps you do not have the source code), then the next

easiest thing to do is to... Slow down the hardware!

Practically all microprocessors can accept an external clock. In most

cases, you just disconnect the crystal and drive one of it's connections

with the external clock. Check the datasheets for the microprocessor that

you are using.

Appendix B describes a small programmable clock generator that can be

used to produce an external clock from 4.0Mhz all the way down to the audio

range. This device is easy to build, and is a very handy companion to PCLA.

Simply connect as an external clock on your target system, and you will

be able to easily adjust the clock speed to exactly suit your sampling

requirements.

.P

> File Formats

The data saved by the F)ile function has the following format:

.f-

2       - Configured Interval between samples (us)

2       - Configured number of samples to capture

1       - Current ending position in preview buffer(+1)

1       - Trigger mask  value for A-E input lines

1       - Trigger match value for A-E input lines

1       - Trigger mask  value for 0-7 input lines

1       - Trigger match value for 0-7 input lines

256     - Preview buffer for A-E input lines *

256     - Preview buffer for 0-7 input lines

2 [A]   - Number of review entries

1 [B]   - Review entry for A-E input lines *

1 [B]   - Review entry for 0-7 input lines

[B] entries repeat for [A] times (0=65536).

* A-E entries are stored in the upper 5 bits of the byte, and

the high bit (bit 7) is inverted.

.f+

.P

> APPENDIX A - Modifying older Parallel Ports

The original IBM Printer Adapter, and many of the early clones were

very easy to modify to become bidirectional. The output driver for these

cards is a 74LS347, which has an -OE line (pin 1) that allows it to be

tristated, effectivly removing it from the circuit, leaving only the

diagnostic "feedback" registers, which just happen to function exactly

as a bidirectional port in input mode.

It also happens, that these ports use a 74LS174 hex latch for the

control signals, and since there are only 5 control signals, there is an

unused latch available. If also happens that the input of this unused

latch is already connected to D5, the right pin for a bidirectional

control signal. The output of this latch (pin 15) is not connected.

All you need to do to make this type of port a bidirectional port

is to cut the trace from pin1 of the 74LS374 which connects it to ground

(always enabled), and connect that pin instead of pin15 of the 74LS174

latch. This causes the port to be configured as an output when D5 of the

control register is written with 0, and as an input when D5 of the

control register is written with 1.

NOTE: If you plan to use the parallel port only with PCLA, and want to

completely avoid the possibility of signal contention due to the output

buffers being switched on, you can simply disconnect pin1 of the 74LC374

from ground and connect it to +5v.. This will permanently remove the

output driver from the circuit, making the port an input only port.

.P

> APPENDIX B - A Programmable Clock Generator

The drawing file PCLOCK.DWG contains a microcad format programmable

clock generator, which can generate clock ranging from 4.00 Mhz down to

the audio range.

This device uses three 74LS163's, two of them as a programmable

divider, and the last one as a flip-flop to obtain a square wave output,

and also provides divide by 2, divide by 4 and divide by 8 outputs, which

can be used to further reduce the clock frequency, or in cases where

synchronized double/half clocks are required.

The project can be built on a small circuit board, and can be powered

from the target system, or any available 5v supply.

The following clock frequencies are available via the DIP switch

setting:

.f-

Mhz     Switch    Mhz     Switch    Mhz     Switch    Mhz     Switch

----------------  ----------------  ----------------  ----------------

0.03125=00000000  0.03571=00000100  0.04167=00000010  0.05000=00000110

0.03137=10000000  0.03587=10000100  0.04188=10000010  0.05031=10000110

0.03150=01000000  0.03604=01000100  0.04211=01000010  0.05063=01000110

0.03162=11000000  0.03620=11000100  0.04233=11000010  0.05096=11000110

0.03175=00100000  0.03636=00100100  0.04255=00100010  0.05128=00100110

0.03187=10100000  0.03653=10100100  0.04278=10100010  0.05161=10100110

0.03200=01100000  0.03670=01100100  0.04301=01100010  0.05195=01100110

0.03213=11100000  0.03687=11100100  0.04324=11100010  0.05229=11100110

0.03226=00010000  0.03704=00010100  0.04348=00010010  0.05263=00010110

0.03239=10010000  0.03721=10010100  0.04372=10010010  0.05298=10010110

0.03252=01010000  0.03738=01010100  0.04396=01010010  0.05333=01010110

0.03265=11010000  0.03756=11010100  0.04420=11010010  0.05369=11010110

0.03279=00110000  0.03774=00110100  0.04444=00110010  0.05405=00110110

0.03292=10110000  0.03791=10110100  0.04469=10110010  0.05442=10110110

0.03306=01110000  0.03810=01110100  0.04494=01110010  0.05479=01110110

0.03320=11110000  0.03828=11110100  0.04520=11110010  0.05517=11110110

0.03333=00001000  0.03846=00001100  0.04545=00001010  0.05556=00001110

0.03347=10001000  0.03865=10001100  0.04571=10001010  0.05594=10001110

0.03361=01001000  0.03883=01001100  0.04598=01001010  0.05634=01001110

0.03376=11001000  0.03902=11001100  0.04624=11001010  0.05674=11001110

0.03390=00101000  0.03922=00101100  0.04651=00101010  0.05714=00101110

0.03404=10101000  0.03941=10101100  0.04678=10101010  0.05755=10101110

0.03419=01101000  0.03960=01101100  0.04706=01101010  0.05797=01101110

0.03433=11101000  0.03980=11101100  0.04734=11101010  0.05839=11101110

0.03448=00011000  0.04000=00011100  0.04762=00011010  0.05882=00011110

0.03463=10011000  0.04020=10011100  0.04790=10011010  0.05926=10011110

0.03478=01011000  0.04040=01011100  0.04819=01011010  0.05970=01011110

0.03493=11011000  0.04061=11011100  0.04848=11011010  0.06015=11011110

0.03509=00111000  0.04082=00111100  0.04878=00111010  0.06061=00111110

0.03524=10111000  0.04103=10111100  0.04908=10111010  0.06107=10111110

0.03540=01111000  0.04124=01111100  0.04938=01111010  0.06154=01111110

0.03556=11111000  0.04145=11111100  0.04969=11111010  0.06202=11111110

.P

Mhz     Switch    Mhz     Switch    Mhz     Switch    Mhz     Switch

----------------  ----------------  ----------------  ----------------

0.06250=00000001  0.08333=00000101  0.12500=00000011  0.25000=00000111

0.06299=10000001  0.08421=10000101  0.12698=10000011  0.25806=10000111

0.06349=01000001  0.08511=01000101  0.12903=01000011  0.26667=01000111

0.06400=11000001  0.08602=11000101  0.13115=11000011  0.27586=11000111

0.06452=00100001  0.08696=00100101  0.13333=00100011  0.28571=00100111

0.06504=10100001  0.08791=10100101  0.13559=10100011  0.29630=10100111

0.06557=01100001  0.08889=01100101  0.13793=01100011  0.30769=01100111

0.06612=11100001  0.08989=11100101  0.14035=11100011  0.32000=11100111

0.06667=00010001  0.09091=00010101  0.14286=00010011  0.33333=00010111

0.06723=10010001  0.09195=10010101  0.14545=10010011  0.34783=10010111

0.06780=01010001  0.09302=01010101  0.14815=01010011  0.36364=01010111

0.06838=11010001  0.09412=11010101  0.15094=11010011  0.38095=11010111

0.06897=00110001  0.09524=00110101  0.15385=00110011  0.40000=00110111

0.06957=10110001  0.09639=10110101  0.15686=10110011  0.42105=10110111

0.07018=01110001  0.09756=01110101  0.16000=01110011  0.44444=01110111

0.07080=11110001  0.09877=11110101  0.16327=11110011  0.47059=11110111

0.07143=00001001  0.10000=00001101  0.16667=00001011  0.50000=00001111

0.07207=10001001  0.10127=10001101  0.17021=10001011  0.53333=10001111

0.07273=01001001  0.10256=01001101  0.17391=01001011  0.57143=01001111

0.07339=11001001  0.10390=11001101  0.17778=11001011  0.61538=11001111

0.07407=00101001  0.10526=00101101  0.18182=00101011  0.66667=00101111

0.07477=10101001  0.10667=10101101  0.18605=10101011  0.72727=10101111

0.07547=01101001  0.10811=01101101  0.19048=01101011  0.80000=01101111

0.07619=11101001  0.10959=11101101  0.19512=11101011  0.88889=11101111

0.07692=00011001  0.11111=00011101  0.20000=00011011  1.00000=00011111

0.07767=10011001  0.11268=10011101  0.20513=10011011  1.14286=10011111

0.07843=01011001  0.11429=01011101  0.21053=01011011  1.33333=01011111

0.07921=11011001  0.11594=11011101  0.21622=11011011  1.60000=11011111

0.08000=00111001  0.11765=00111101  0.22222=00111011  2.00000=00111111

0.08081=10111001  0.11940=10111101  0.22857=10111011  2.66667=10111111

0.08163=01111001  0.12121=01111101  0.23529=01111011  4.00000=01111111

0.08247=11111001  0.12308=11111101  0.24242=11111011  0 (DC) =11111111

.f+

This device should be built with the packaged oscilator located in a

socket, allowing it to be changed to obtain even more clock frequencies.

Just scale the above chart accordingly.

.P

> APPENDIX C - Making a clean boot disk

To make a DOS boot disk, you need a DOS or Windows9x system. You

cannot make a DOS boot disk from a WindowsNT system (although you can

still boot the disk).

If you are running Windows95 OSR2 or later, and need to be able to

access your hard drive from the booted floppy, then you should make your

boot disk from the Windows DOS prompt.

The steps in making a boot disk are:

.f-

1) Insert a blank floppy disk into your A: drive.

ANY data already on this disk will be lost!

2) Get to the DOS prompt.

If you are running Windows3.1, exit to the DOS prompt.

If you are running Windows9x, activate the "MS-DOS Prompt"

item on the start menu/Programs.

3) At the DOS prompt, enter the command: FORMAT A: /S /U

/S tells FORMAT to put a bootable SYSTEM on the disk

/U tells format to do a complete (uncondional) format

* If you are running an older version of DOS, FORMAT may not

accept the '/U' option, in which case, just leave it off,

as older DOS versions always do an unconditional format.

4) Position to the directory where PCLA is installed.

Assuming that you installed to the default: C:\PCLA,

use the command: CD C:\PCLA

5) Copy the PCLA.COM program onto the floppy disk:

use the command: COPY C:PCLA.COM A:

6) If you want any other commands on your disk, copy them on

at this time. DOS usually stores it commands in "C:\DOS",

while Windows9x puts them in "C:\WINDOWS\COMMAND".

If you don't know where a particular command is located, try

typing 'PATH'. This will show you all of the directories

where the system searches for commands. You may have to go

through them one by one to find the command.

.f+

Your BOOT disk is now complete. You should be able to shut down your

computer, insert the FLOPPY disk into your A: drive, and reboot into

a clean copy of DOS. If you are prompted for the TIME and DATE when

the system boots, just press ENTER. You will receive a DOS prompt, and

you can run PCLA, or any other commands that you copied onto the disk.

.P

If your system boots from the hard drive, even with the BOOT disk in

drive A:, then the most likely cause is that your system BIOS is

configured to boot directly from the hard drive.

You may have to consult your system documentation to change the boot

type, however generally, you will have to:

.f-

1) Watch the computer startup screen closely when it is first

turned on... Usually there will be a prompt to press a certain

key to enter the BIOS setup/configuration. Often this key is

DEL, but sometimes it is something else.

2) When prompted, press the KEY. This should get you into the

BIOS setup screens.

3) Look for an item with a name something like "BIOS features" or

"BIOS options" and select it.

4) Look for any options which indicate how the system boots. It

will be called "System BOOT", "System BOOT sequence", "Boot

order" or something similar. It will most likely be set to

some option in which C appears before A, such as "C only",

"C,A", "CDROM,C,A", "C then A" or something similar.

5) Find the keys to change the options, and change the BOOT option

to "A,C" or another suitable option in which "A" appears before

"C". Make sure that "C" is mentioned in the option, or else you

system may not boot from the hard drive at all (you can always

come back here and fix it if this happens).

DO NOT change any other options in the BIOS setup screens!

6) Exit the "BIOS options" screen, and find the key to "Save and

Exit Setup" (or something similar).

.f+

Once you have saved the new settings, your system should BOOT from the

floppy disk if one is present in the A: drive when the system is turned

on or reset.